LCS3172: 3172 Lan Channel Station for TCP/IP support

=====================================================

LCS3172 emulates a 3172 LAN Channel Station and provides the mechanism

used by mainframe TCP/IP products to interface to Local Area Networks.

DEVICE TYPE: 3088 (even/odd address pair)

===========

CONFIGURING LCS3172

===================

To configure an LCS3172, go to the Configurator "Update System Devices"

(F2) screen and configure a pair of 3088 device addresses. The first address

must be even. Use the manager code for LCS3172 in the Mgr field.

In the FileName/Parameters field put the corresponding adapter device name

you are going to use and the type of connection you want (not required on token

ring).

Example:

If you want to configure a standard ethernet interface using the ethernet

adaptor, then configure your device under the configuration as follows.

----------------------------------------------------------------------------

Addr Device Label  Atype  Size     Mgr  FN/P                 PATH:

--------------------------------------- ------------------------------------

>     >       >     >         >A >

600  3088                          A  /dev/ent0,standard

601  3088                          A  /dev/ent0,standard

----------------------------------------------------------------------------

MGR Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=AWS3172  A=LCS3172  B=AWS2703  C=AWSICA   D=AWSTAPE

600 and 601 are the addresses, "A" is the LCS3172 manager code, and

the device type is 3088.

Example:

If you want to configure a IEEE 802.3 ethernet interface using the ethernet

adaptor, then configure your device under the configuration as follows.

----------------------------------------------------------------------------

Addr Device Label  Atype  Size     Mgr  FN/P                 PATH:

--------------------------------------- ------------------------------------

>     >       >     >         >A >

600  3088                          A  /dev/ent0,802.3

601  3088                          A  /dev/ent0,802.3

----------------------------------------------------------------------------

MGR Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=AWS3172  A=LCS3172  B=AWS2703  C=AWSICA   D=AWSTAPE

Example:

If you want to configure a token ring interface using the token ring

adaptor, then configure you device under the configuration as follows.

----------------------------------------------------------------------------

Addr Device Label  Atype  Size     Mgr  FN/P                 PATH:

--------------------------------------- ------------------------------------

>     >       >     >         >A >

600  3088                          A  /dev/tok0

601  3088                          A  /dev/tok0

----------------------------------------------------------------------------

MGR Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=AWS3172  A=LCS3172  B=AWS2703  C=AWSICA   D=AWSTAPE

In all the above examples the adapter device name used assumes the network

adaptor is recognized by AIX as ent0 or tok0. However any AVAILABLE tok or ent

number can be used (ie. tok1, ent2 etc).

Each LCS3172 address pair must be sequential even/odd addresses (even is read

address and odd is write address). Only one LAN device can be specified per

pair.

Multiple pairs with unique LAN devices are supported, however a max limit of

2 is recommended.  Multiple pairs correspond to multiple DEVICE and LINK

statements in the TCPIP PROFILE (see examples below).

Multiple LANs (LINKs) on a single pair of (DEVICE) addresses is NOT supported.

LCS3172 can coexist with LAN3172 on the same LAN.

INSTALLATION NOTES:

==================

NOTE: Starting at r390.obj 4.2.1.5, modifications to ipl390 are NOT needed.

LCS3172 is started by changing the "nostart" keyword tp "noparms" on the

"Start Device Manager Parameter Input/Display" panel of the Configurator.

Prior to r390.obj 4.2.1.5, uncomment the lcs3172 section in the

/usr/bin/r390/ipl390 script.

AIX CONFIGURATION:

=================

Since LCS3172 MUST use the DIRECT LAN INTERFACE, and because LLC Direct

Network service allows only one application at a time to use the DIRECT

network service, the LAN used by LCS3172 cannot be simultaneously shared

with other applications that also use the DIRECT INTERFACE (eg. AIX TCP/IP).

** NOTE:

**   If you want to have AIX TCP/IP installed simultaneously with the

**   LCS3172 support, then you MUST have two LAN adapters installed --

**   one for AIX TCP/IP and one for System/390 TCP/IP via LCS3172.

*------------------------------------------------------------------------------*

|  R/390 RELEASES 4.1.0.0 and LOWER:                                           |

|                                                                              |

|  Prior to release 4.1.0.1 LCS3172 requires the COMIO interface to be active. |

|  This means that 1) your adapter has to support COMIO, and 2) you must have  |

|  the COMIO compatibility filesets installed (required on AIX 4.1).           |

|  The COMIO filesets are called bos.compat.lan.                               |

|                                                                              |

|  To verify that the COMIO interface is enabled, check to see that the        |

|  /dev/tok0 or /dev/ent0 path exists.                                         |

|                                                                              |

|  Note: On releases of r390.obj 4.1.0.0 and lower, the link_number parameter  |

|        on the TCP/IP LINK statement must be 1.  See below...                 |

*------------------------------------------------------------------------------*

*------------------------------------------------------------------------------*

|  R/390 RELEASES 4.1.0.1 and HIGHER:                                          |

|                                                                              |

|  Starting at release 4.1.0.1, LCS3172 uses the DLPI interface. This allows   |

|  LCS3172 to work with virtually any IBM token ring or ethernet adapter.      |

|  However, this interface MUST be enabled by activating the /etc/pse.conf     |

|  file:                                                                       |

|                                                                              |

|  1) First, comment the dlpi statements in /etc/dlpi.conf by inserting the #  |

|     character in the first column.                                           |

|                                                                              |

|   #d+      dlpi    en      /dev/dlpi/en    # streams dlpi ethernet driver    |

|   #d+      dlpi    et      /dev/dlpi/et    # streams dlpi 802.3 driver       |

|   #d+      dlpi    tr      /dev/dlpi/tr    # streams dlpi token ring driver  |

|   #d+      dlpi    fi      /dev/dlpi/fi    # streams dlpi FDDI driver        |

|                                                                              |

|  2) Next, remove the comment character from the dlpi statements in           |

|     /etc/pse.conf                                                            |

|                                                                              |

|   d+      dlpi    en      /dev/dlpi/en    # streams dlpi ethernet driver     |

|   d+      dlpi    et      /dev/dlpi/et    # streams dlpi 802.3 driver        |

|   d+      dlpi    tr      /dev/dlpi/tr    # streams dlpi token ring driver   |

|   d+      dlpi    fi      /dev/dlpi/fi    # streams dlpi FDDI driver         |

|                                                                              |

|  3) Enter these streams load commands at the AIX prompt:                     |

|                                                                              |

|     strload -u -f /etc/dlpi.conf                                             |

|     strload -f /etc/pse.conf                                                 |

|                                                                              |

|  4) Reboot AIX (shutdown -Fr)                                                |

|                                                                              |

|  To verify that the DLPI interface is enabled, check to see that the         |

|  /dev/dlpi/tr, /dev/dlpi/en, and /dev/dlpi/et paths were created.            |

*------------------------------------------------------------------------------*

LCS3172 will set up the a network definition on the target adapter

under the control of the Operating System's TCP/IP component, therefor it

is not necessary to set up a network definition from AIX. If an IP network

is defined from AIX, then it must be "detached" before IPLing the

Operating System.

This is automated in release 4.2.1.5 and up. Statements are generated and

inserted during the ipl process.

For prior releases, detach a define IP network from the target adapter

using smit/ty as follows.  For example, to detach en0 from the

standard ethernet interface configured on adaptor 0, enter smit/ty:

Communications Applications and Services

TCP/IP

Further Configuration

Network Interfaces

Network Interface Selection

Change/Show Characteristics of a Network Interface

en0   Standard Ethernet Network Interface

change the CURRENT STATE to 'up'

Do

change the CURRENT STATE to 'detach'

Do

This can also be automated in earlier releases by adding the appropriate

chdev statements in the ipl script.  In this example, insert these 2

statements in the LCS3172 section:

chdev -l en0 -a state=up

chdev -l en0 -a state=detach

SUMMARY CHECK:

==============

The Adapter type (IBMTR, ETHERNET, or 802.3) specified in the VM/TCPIP

Configuration, and the device specified in the FN/P field of the Configurator

(eg. /dev/tok0 for token ring, /dev/ent0,standard for standard ETHERNET,

and /dev/ent0,802.3 for 802.3 ETHERNET Adapter) must match.  And it

is also very important that the specified device must be in 'detach'

status in AIX.  This is done automatically in r390.obj 4.2.1.5 and up.

S/390 Operating System TCP/IP CONFIGURATION:

===========================================

See your TCP/IP Customization and Installation Guide for proper configuration

of the TCP/IP component of your operating system.  The following is an

example only, however in all cases, the adapter number in third parameter

on the LINK statement referred to as the "link_number" must always be 0.

Note: On releases of r390.obj 4.1.0.0 and lower, the link_number parameter

on the TCP/IP LINK statement must be 1.  This was corrected in 4.1.0.1.

This is the definition of a Token-Ring adapter in PROFILE TCPIP:

#  Hardware Definitions

#  3172 @ 600/601 TokenRing

DEVICE UNITY LCS 600      /* 600 is the Even (READ) address */

LINK TR600 IBMTR 0 UNITY

And this is the PROFILE TCPIP definition of an Ethernet adapter:

#  Hardware Definitions

#  3172 @ 600/601 Ethernet

DEVICE UNITY LCS 600 	/* 600 is the Even (READ) address */

LINK ETHER600 ETHERNET 0 UNITY

EXAMPLE OF LCS3172 CONFIGURED WITH 2 LANS

=========================================

In my example, tok0 is our site LAN, tok1 is a private LAN.

if a PRIMARYINTERFACE statement is not coded, the first LAN

specified in TCPIP PROFILE becomes the PRIMARY LAN, so specify

your preferred LAN first.  LCS3172 matches the first pair in

the devmap, to the first LINK in TCPIP PROFILE, second pair

with the second, and so on up to 4 pairs (ie. 4 LANs).

Example of Device Map:

Addr  Device           MGR FN/P

----  -------          --- -----------

044A  3088              8  /dev/tok0

044B  3088              8  /dev/tok0

044C  3088              8  /dev/tok1

044D  3088              8  /dev/tok1

Example of Relevant TCPIP PROFILE Entries:

DEVICE UNIT0 LCS 44A

LINK TR44A IBMTR 0 UNIT0

DEVICE UNIT1 LCS 44C

LINK TR44C IBMTR 1 UNIT1

HOME

9.117.118.178

9.67.174.203

GATEWAY

9      =    TR44A  DEFAULTSIZE   0.255.255.0    0.117.118.0

9      =    TR44C  DEFAULTSIZE   0.255.255.0    0.67.174.0

DEFAULTNET 9.117.118.251    TR44A   DEFAULTSIZE  0.255.255.0    0

START UNIT0

START UNIT1

Example of Relevant TCPIP DATA Entries:

HOSTNAME CASTALDO1

DOMAINORIGIN  POK.IBM.COM

NSINTERADDR  9.117.118.254