AS/400 PRINT FUNCTIONS FOR THE IBM 4247 PRINTER

_______________________________________________

IBM's 4247 heavy duty, dot-matrix printer is capable of printing 700 charac-

ters per second and offers great flexibility in forms handling.  The standard

push tractor and manual cut sheet paper paths can be augmented with an

optional second tractor path or a three-input bin Auto Sheet Feed (ASF)

device.  The second tractor can be used for a second set of forms, or it can

be combined into a single push/pull path for more challenging print jobs.

Envelopes (USA #6 and #10 sizes) are supported via the manual cut sheet feed

and the ASF (bin 1).

The 4247 can automatically switch between the parallel and serial ports.

Through these ports the printer can be set up to emulate either an IBM Pro-

printer XL III, an Epson FX-1050, or an IBM 2381 printer, or placed in native

4247 mode.  Datastream commands unique to the 4247 are supported in each of

the emulation modes.

ASCII ATTACHMENT

________________

The 4247 can attach to the AS/400 through numerous emulators such as the

ASCII Work Station Controller or InfoWindow displays.  Each of these attach-

ments emulate an SCS (host) printer and convert the SCS datastream to that

required by the ASCII device.  As such, the print functions support are

dependent on the attachment emulator.

Host Print Transform (HPT) was added in OS/400 V2R3 to provide consistent

support for ASCII printers across most attachments.  HPT delivers the richest

set of print functions regardless of the SCS printer emulated.  With Host

Print Transform enabled, the AS/400 generates an ASCII datastream that is

passed transparently through the attaching emulator to the printer.

While the emulators will support the 4247 to the same extent as they sup-

ported the printers it emulates, no enhancements are planned.  Support for

4247-unique commands are limited to Host Print Transform.  The AS/400 can

support the selection of up to three of the paper sources featured by the

4247.  Several new customization objects are provided with the printer

which support various configurations of forms handling devices.

IBM4247M:  Object IBM4247M is designed for use with a single tractor feed

installed on the printer or to allow manual selection of the paper source.

No drawer select commands will be issued by the host, so that they do not

override the manual selection.  This object allows the use of all input

sources.

IBM4247:  Object IBM4247 is designed for use with two tractor feeds installed

on the printer and host-selection between the feeds is desired.  It will park

the forms in the front paper path and load those in the rear paper path when

drawer 2 is specified.  Conversely, specifying drawer 1 will park the rear

forms and load the front forms.

However, a number of restrictions on printer file parameters apply when uti-

lizing this object. Refer to the "Attachment Notes: Dual Continuous Form"

section, immediately following the matrix, for more information. For this

reason, The IBM4247M object is recommended for most installations.

IB4247A3:  Object IB4247A3 is designed for use with the optional Auto Sheet

Feed unit.  Drawers 1, 2, and 3 correspond to bin 1, 2, and 3 on the ASF.

Specifying an envelope feed (*E1) will also select bin 1.  This object is

also designed to reduce the pitch of output generated with AS/400 print

defaults which assume a 13.2 inch wide page size.

IB4247A2:  Object IB4247A2 is identical to object IB4247A3 with the third

drawer select tag removed.  This tag is not supported by HPT prior to OS/400

V3R1.  Drawers  1 and 2 correspond to bins 1 and 2 on the ASF.

IB4247X3:  Object IB4247X3 is designed for use with a combination of ASF bins

and tractor feeds.  Drawer 2 selects the front tractor feed.  Drawers 1 and 3

correspond to bin 1 and 2 on the ASF.  Specifying an envelope feed (*E1) will

also select bin 1.

IB4247X2:  Object IB4247X2 is identical to object IB4247X3 with the third

drawer select tag removed.  This tag is not supported by HPT prior to OS/400

V3R1. Drawer 1  corresponds to bin 1 on the ASF. Drawer 2 selects the front

tractor feed.

As the name implies, these objects can be customized to select any combina-

tion of input sources.  Limited support can be attained for additional

sources.  For example, the manual feed tags are not assigned in the objects

since the printer datastream does not support the selection of the manual

cutsheet feeder.  They could be used to select another forms path.  Also, if

ASF bin 1 will always be used for envelopes, drawer 1 could be directed to

another paper source.

AS/400 CONFIGURATION SUPPORT

Prior to configuring the printer, the appropriate W/S customization object

must be created.  The source text files for four objects described in the

previous section are included on the driver diskette shipped with the

printer.  The header of each file contains instructions for creating

customization object from them.  The configuration instructions below assume

that the objects were given the name of the source file.

Alternatively, one may create the source files manually by retrieving the

customization object for the IBM 2381 and adding the appropriate forms han-

dling commands. Refer to section 1.3 for more information

The AS/400 device configurations can be created or changed manually using the

CRTDEVPRT command or CHGDEVPRT command.  The configuration values needed

depend on whether the printer is attached to the ASCII Work Station Con-

troller and which forms handling features are installed on the printer.

Attachment Emulator

Specify the keyword values in the set below which corresponds to the printer

attachment utilized.

ASCII WSC ATTACHMENT:  AS/400 Device Configuration (CRTDEVPRT)

o   Device Type (TYPE): 5204

o   Model (MODEL): 1

o   Emulated twinaxial device (EMLDEV): 3812

o   Emulated ASCII device (EMLASCII): *YES

o   Host Print Transform (HPT): *YES

NON-ASCII WSC ATTACHMENT:  AS/400 Device Configuration (CRTDEVPRT)

o   Device Type (TYPE): 3812

o   Model (MODEL): 1

o   Host Print Transform (HPT): *YES

Note:  Some attachments may not provide a 3812 (5219) printer emulation.

Specify the type and model supported by the attachment emulator. With HPT,

identical functionality is achieved.

Forms Handling Devices

Specify the keyword values in the set below which corresponds to the forms

handling features installed on the printer.

SINGLE CONTINUOUS FORMS FEED:  AS/400 Device Configuration (CRTDEVPRT)

o   Manufacture Type and Model (MFRTYPMDL): *WSCST

o   Paper source 1 (PPRSRC1): *CONT132

o   Paper source 2 (PPRSRC2): *NONE

o   Workstation Customization Object (WSCST): IBM4247M

or...

o   Manufacture Type and Model (MFRTYPMDL): *IBM2381

MANUAL SELECTION OF PAPER FEEDS:  AS/400 Device Configuration (CRTDEVPRT)

o   Manufacture Type and Model (MFRTYPMDL): *WSCST

o   Paper source 1 (PPRSRC1): *NONE

o   Paper source 2 (PPRSRC2): *NONE

o   Workstation Customization Object (WSCST): IBM4247M

or...

o   Manufacture Type and Model (MFRTYPMDL): *IBM2381

DUAL CONTINUOUS FORMS FEED:  AS/400 Device Configuration (CRTDEVPRT)

o   Manufacture Type and Model (MFRTYPMDL): *WSCST

o   Paper source 1 (PPRSRC1): *LETTER or *A4 (not *contxxx, or *none)

o   Paper source 2 (PPRSRC2): *NONE

o   Workstation Customization Object (WSCST): IBM4247

COMBINATION OF AUTO SHEET FEED (ASF) BINS AND TRACTER FEEDS:  AS/400 Device

Configuration (CRTDEVPRT)

o   Manufacture Type and Model (MFRTYPMDL): *WSCST

o   Paper source 1 (PPRSRC1): *LETTER  (actual form size ASF bin 1)

o   Paper source 2 (PPRSRC2): *NONE    (*CONTxxx cannot be specified)

o   Workstation Customization Object (WSCST):

-   OS/400 V2R3 or V3R0M5: IB4247X2

-   OS/400 V3R1: IB4247X3

AUTO SHEET FEED (ASF):  AS/400 Device Configuration (CRTDEVPRT)

o   Manufacture Type and Model (MFRTYPMDL): *WSCST

o   Paper source 1 (PPRSRC1): *LETTER (actual form size)

o   Paper source 2 (PPRSRC2): *LETTER (actual form size)

o   Workstation Customization Object (WSCST):

-   OS/400 V2R3 or V3R0M5: IB4247A2

-   OS/400 V3R1: IB4247A3

Note:  If the *IBM2381 value for MFRTYPMDL is used in place of the supplied

4247 drivers, some fonts featured by the 4247 will not be selectable via the

host datastream.

ATTACHMENT EMULATOR SETUP

Refer to setup instructions provided with emulator.

4247 SETUP

The IBM4247 customization tables were designed for use with the IBM 4247 in

either native or 2381 emulation mode.

AS/400 PTF REQUIREMENTS

The IBM 4247 printer associates several print parameters with the paper

source.  The op panel allows the user to set default values for these param-

eter on each source.  Switching paper sources causes the default values to be

used until the appropriate datastream command is receive.  The following

PTF's are required to force HPT to resend the CPI and print quality parame-

ters after all paper source selection commands.

o   V2R3     SF27481

o   V3R0M5   SF27482

o   V3R1     SF27509

IBM 4247 PRINT FUNCTIONS MATRIX

_______________________________

The results documented were obtained with the following device microcode

levels.

o   Base  : 1.11

+---------------------------------------------------------------------------+

| Table 1    -----       AS/400 Print Functions for the IBM 4247 Printer    |

+---------------+--------------+--------------+--------------+--------------+

| FUNCTION      |      HPT     |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

|                           OFFICE PRINT FUNCTIONS                          |

|                                                                           |

+---------------+--------------+--------------+--------------+--------------+

| Bold          | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Change char-  |              |              |              |              |

| acter         |              |              |              |              |

| set/code page |              |              |              |              |

| ID            |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Change font   |              |              |              |              |

| (type style)  | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Color         |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Double-byte   |              |              |              |              |

| character set |              |              |              |              |

| (DBCS) data   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Double-sided  |              |              |              |              |

| printing      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Double-sided  |              |              |              |              |

| tumble        |              |              |              |              |

| printing      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Graphics      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Image         |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Justification |              |              |              |              |

| - Half (50%)  | X(5)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Justification |              |              |              |              |

| - Right       |              |              |              |              |

| (100%)        | X(5)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Overstrike    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Labels - Con- |              |              |              |              |

| tinuous Form  | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Labels -      |              |              |              |              |

| Sheet Feed    | X(1)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (any     |              |              |              |              |

| value from    |              |              |              |              |

| 1.00-1440.00) |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (2.54)   | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (4.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (5.33)   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (6.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (8.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (9.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (9.6)    |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (12.0)   | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (24.0)   | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Lines per     |              |              |              |              |

| inch (48.0)   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (0.5)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (1.0)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (1.5)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (2.0)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (2.5)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Line spacing  |              |              |              |              |

| (3.0)         | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Paper Length  |              |              |              |              |

| in Inches     |              |              |              |              |

| (Max)         | 22.75        |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Print Line in |              |              |              |              |

| Inches (Max)  | 13.20        |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Paper source  |              |              |              |              |

| - continuous  |              |              |              |              |

| feed          | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Paper source  |              |              |              |              |

| - envelope    |              |              |              |              |

| feed          | X(1)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Paper source  |              |              |              |              |

| - manual      |              |              |              |              |

| sheet/envelope|              |              |              |              |

| feed          | See note 2.  |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Paper source  |              |              |              |              |

| - paper       |              |              |              |              |

| drawer        |              |              |              |              |

| (maximum)     | 3(1)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Print quality |              |              |              |              |

| - Draft       | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Print quality |              |              |              |              |

| - Letter      | X(3)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Print quality |              |              |              |              |

| - Text        |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Required      |              |              |              |              |

| Backspace     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Rotation -    |              |              |              |              |

| automatic     |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Rotation - 90 |              |              |              |              |

| degrees       |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Rotation -    |              |              |              |              |

| 180 degrees   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Rotation -    |              |              |              |              |

| 270 degrees   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Special char- |              |              |              |              |

| acters (such  |              |              |              |              |

| as: cent      |              |              |              |              |

| sign, one     |              |              |              |              |

| half, and     |              |              |              |              |

| section       |              |              |              |              |

| symbol)       | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Subscript/    | X            |              |              |              |

|  Superscript  |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Symbols (Code |              |              |              |              |

| Page 259)     | X(4)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Tabs          | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers 1-65  |              |              |              |              |

| (10-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 132          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 66-153        |              |              |              |              |

| (12-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 158          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 154-200 (pro- |              |              |              |              |

| portional)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 201-210       |              |              |              |              |

| (13.3-pitch)  |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 211-239       |              |              |              |              |

| (15-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 198          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 240-246       |              |              |              |              |

| (5-pitch)     |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 66           |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 247-257 and   |              |              |              |              |

| 274-279       |              |              |              |              |

| (17.1-pitch)  |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 220          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 258-259       |              |              |              |              |

| (18-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 260-273       |              |              |              |              |

| (8.55-pitch)  |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 105          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 280-284       |              |              |              |              |

| (20-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          | 254          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 285-289       |              |              |              |              |

| (25-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 290-299       |              |              |              |              |

| (27-pitch)    |              |              |              |              |

| maximum char- |              |              |              |              |

| acters per    |              |              |              |              |

| line          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Typestyle/font|              |              |              |              |

| numbers       |              |              |              |              |

| 300-65534     |              |              |              |              |

| (typographical|              |              |              |              |

| and user-     |              |              |              |              |

| defined       |              |              |              |              |

| fonts)        |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Underline     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Zero index    |              |              |              |              |

| carrier       |              |              |              |              |

| return (ZICR) | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

|                   SYSTEM PRINT (PRINTER FILE) FUNCTIONS                   |

|                                                                           |

+---------------+--------------+--------------+--------------+--------------+

| Double-sided  |              |              |              |              |

| printing      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Double-sided  |              |              |              |              |

| tumble        |              |              |              |              |

| printing      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font (*CPI)   |              |              |              |              |

| CPI (5.0)     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font (*CPI)   |              |              |              |              |

| CPI (10.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font (*CPI)   |              |              |              |              |

| CPI (12.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font (*CPI)   |              |              |              |              |

| CPI (15.0)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font (*CPI)   |              |              |              |              |

| CPI (16.7)    | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| FORMFEED(*CONT) X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| FORMFEED(*CUT)| See note 2   |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| FORMFEED(AUTOCUT)            |              |              |              |

| DRAWER(n)     | 3(1)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| FORMFEED(AUTOCUT)            |              |              |              |

| DRAWER(*E1)   | X(1)         |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Hardware Jus- |              |              |              |              |

| tification 50 | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Hardware Jus- |              |              |              |              |

| tification    |              |              |              |              |

| 100           | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| DBCS/IGC Data |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| LPP (11" @ 6  |              |              |              |              |

| LPI) cont/cut | 66/64        |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| LPP (11" @ 8  |              |              |              |              |

| LPI) cont/cut | 88/86        |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(*AUTO) |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(*COR)  |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(0)     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(90)    |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(180)   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PAGRTT(270)   |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PRTQLTY(*NLQ) | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PRTQLTY(*STD) |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PRTQLTY(*DRAFT) X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| PRTQLTY(*FASTDRAFT)          |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| RPLUNPRT      | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

|                            OTHER DDS FUNCTIONS                            |

|                                                                           |

+---------------+--------------+--------------+--------------+--------------+

| Chrid         |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Chrsiz        |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Highlight     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Skipa/skipb   | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Spacea/Spaceb | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Underline     | X            |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

|                       OTHER ADVANCED PRINT FUNCTIONS                      |

|                                                                           |

+---------------+--------------+--------------+--------------+--------------+

| Barcode       |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| BOX, LINE,    |              |              |              |              |

| POSITION      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Color         |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Forms Data    |              |              |              |              |

| Mrg           |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Font Download |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Graphics      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Multi-up (0   |              |              |              |              |

| Rot)          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Multi-up (90  |              |              |              |              |

| Rot)          |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| Text Rotate   |              |              |              |              |

| (TXTRTT)      |              |              |              |              |

+---------------+--------------+--------------+--------------+--------------+

| (1) Automatic Sheet Feeder (ASF) option is required.  Host drawer         |

|    selection is limited to a maximum of 2 drawers prior to OS/400 V3R1.   |

| (2) Manual sheet feed is not host (datastream) selectable.  It can be     |

|    manually selected by inserting a form while the printer is not ready.  |

| (3) Near-letter (NLQ).                                                    |

| (4) Only those symbols in the printer resident code pages print correctly |

|    (47 of 191).                                                           |

| (5) Justification of proportional space fonts not supported.              |

+---------------------------------------------------------------------------+

ATTACHMENT NOTES

General Notes

MANUAL CUTSHEET FEED:  The manual cutsheet feeder is not datastream select-

able.  It is activated by inserting a form while the printer in offline.  Any

form loaded at this time will be parked automatically by the printer.

As a result, jobs destined to be printed on manually inserted form may wish

to utilize the FORM or similar keyword to alert the operator to insert the

required form prior to the job being sent from the AS/400.  AS/400 jobs that

specify *CUT for the formfeed parameter will be sent to the presently active

paper source.

PRINT QUALITY:  Host selection of print quality is limited to two of the

three qualities featured by the device.  Specification of *NLQ, *STD, and

*FASTDRAFT result in the highest quality.  Specification of *DRAFT results in

the lowest quality. If the print speed is a high priority and a large number

of print applications use the default *STD quality, it may be advantageous to

edit the customization object instead of modifying every print application.

Refer to section 1.3.1.4 for instructions on modifying the default print

quality.

OCR FONT USE:  Each object provides for the selection of OCR A and B fonts by

specifying FGID (Font Global ID) of 19 and 3, respectively.  Due to

datastream limitations, switching from an to OCR font to any other font

within a job will cause the default print quality to be selected.  This

quality can be specified via the printer op panel. If desired, the

customization object can also be modified to specify the print quality used

when switching from an OCR font.

Dual Continuous Feed

SWITCHING FORMS DEVICES:  The IBM4247 workstation customization object is

designed to provide datastream selection between two continuous forms paths

via the drawer parameter.  Selection of one path will automatically park the

forms from the other path.  In parking the forms, the printer will attempt to

back the forms from beneath the platen.  If the previously printed forms

forms have not been removed prior to switching paths, undesirable results

(paper jams, overprinting, etc.) are likely to occur.  As such, printing on

two different continuous forms cannot be considered an "unattended opera-

tion". Again, one can use the FORM keyword to prompt the operator to remove

the print outs.

PAPER SOURCE 1 VALUE:  Host selection of multiple paper sources is disabled

when the printer is configured as a continuous feed printer.  For devices

configured to use Host Print Transform, the host uses the Paper Source 1

parameter (keyword PPRSRC1) to make this determination.  Therefore, to allow

host selection between the dual continuous form feed, values of *CONT80,

*CONT132, or *NONE cannot be specified.  The user may specify any of the

other special values provided for this parameter.  When possible, choose a

size with a length equal to or lesser than that of the continuous forms used.

As a result, the host is likely to be misinformed as to the actual size of

the forms. This deception is not without consequesences.  These are described

below.  The user must weigh these restrictions against the convenience of

host drawer selection for their application.

Computer Output Reduction:  For print jobs which specify a page size that is

wider or longer than the configured paper size (the PPRSRC1 value) and use

the default value of *AUTO for page rotation, the host will take liberties to

modify the job so that it will fit on the page.  To prevent this and to force

the host to use the values specified in the job, two criterea must be met.

1.  PAGRTT parameter on the printer file must be set to zero (0).

2.  The W/S Customization object must contain a PAGSIZE tag whose length and

width values match those specified in the PAGSIZE parameter of the

printer file (job).

The objects supplied with the printer, except IB4247Ax.TXT, contain tags for

form sizes 13.2"x11", 13.2"x8.5", 8"x11", and 8"x8.5".  The W/S Customization

object can be easily modified to include additional sizes.

If either of the two criterea listed above cannot be met, the following mod-

ification may be made to the job parameters.

Incorrect Page Length One of the actions will be to use the page length of

the configured paper size in place of that specified by the datastream.  If

they happen to be equal (quite common with 11" or A4-sized forms)--no

problem.

Reduced Line Spacing:  If the page length specified is greater than that con-

figured, the host will reduce the spacing between the lines.

Font Selection:  Another action the host will take is to reduce the specified

pitch.  This is mitigated somewhat in the IBM4247 and IBM4247M objects by

setting the CPICOR tags to select the same pitch.  However, if one of the OCR

fonts, or another font for which the user added an individual font tag to the

object, is specified, it will not be selected.

VERIFY ALIGNMENT:  Since the paper source parameter for the dual continuous

form configuration must not be set to *CONT132 or *CONT80, the AS/400 will

not provide an oppurtunity to align the forms.  If this feature is important,

the QWP4019 program can be called with the *CNT parameter.  Refer to the

"AS/400 Printer Device Programming" manual (SC41-3713) for more information.

WORKSTATION CUSTOMIZATION OBJECT SOURCE

_______________________________________

The HPT W/S Customization objects for the IBM 4247 are based on the object

generated for the IBM 2381 printer.  If your version of Operating System/400

does not recognize the *IBM4247 or *IBM4247ASF values for the MFRTYPMDL

keyword, you will need to generate an object manually.  Source code for these

objects are provided with the printer and from the PIP requester tool.

While most AS/400 are connected to LANs or other networks, there may be cir-

cumstances which make it difficult to transfer these source files to the

AS/400.  An alternative approach is to modify a similar object with the tags

listed in the following sections.  To proceed:

1.  Retrieve the IBM 2381 customization object by issuing the following

command:

RTVWSCST DEVTYPE(*TRANSFORM) MFRTYPMDL(*IBM2381) SRCMBR(IBM4247)

2.  Edit resulting file member by adding the customization tags following the

line containing the ":TRNSFRMTBL." tag.

3.  Create new customization object by issuing the following command:

CRTWSCST WSCST(IBM4247)

4.  Create or change printer device description to specify the library and

name of the resulting object in the Workstation Customization Object

(WSCST) parameter:

CHGDEVPRT DEVD(device_name) TRANSFORM(*YES) WSCST(*LIBL/IBM4247)

For additional information concerning Host Print Transform and workstation

customization objects, refer to "AS/400 Printer Device Programming"

(SC41-3713) or "AS400 Workstation Customization Function Programmer's Guide

(SC41-0056).

W/S CUSTOMIZATION SOURCE

Forms Control - Dual Continuous Forms

To allow the datastream (host) to select between two continuous forms devices

based on the DRAWER parameter, add the following lines to the retrieved W/S

Customization objects.

/* Drawer 1 Selection */

:DWRSLT

DRAWER=DRAWER1

DATA='14141B5400'X.

/* Drawer 2 Selection */

:DWRSLT

DRAWER=DRAWER2

DATA='14141B4E00'X.

Forms Control - Auto Sheet Feed

To allow the datastream (host) to select between three input bins of the

optional Auto Sheet Feed (ASF) device based on the DRAWER parameter, add the

following lines to the retrieved W/S Customization objects.

/* Drawer Selection */

/* Envelope Selection */

:DWRSLT

DRAWER=ENVELOPE

DATA='14141B38'X.         /* ASF bin 1 (envelopes )*/

/* Drawer 1 Selection */

:DWRSLT

DRAWER=DRAWER1

DATA='14141B38'X.         /* ASF bin 1 */

/* Drawer 2 Selection */

:DWRSLT

DRAWER=DRAWER2

DATA='14141B39'X.         /* ASF bin 2 */

/* Drawer Selection - Third Drawer */

:DWRNBR

VAROFFSET=2

VARLEN=1

VARTYPE=HIGHLOW

DATA='14141B34'X.         /* ASF bin 3 */

Note: The DWRNBR customization tag is not supported by HPT prior to V3R1.

Additional pitch selection

To allow the datastream (host) to select additional print densities (pitches)

based on the FONT parameter, add the following lines to the retrieved W/S

Customization objects.

/* Font Groups */

:FNTGRP.

/* 10 CPI */

:FNTGRPE

MINFID=1

MAXFID=65

FNTSTR='1B570012'X

FNTEND=''X

FNTWTH=''X.

/* 12 CPI */

:FNTGRPE

MINFID=66

MAXFID=153

FNTSTR='1B5700121B3A'X

FNTEND=''X

FNTWTH=''X.

/* PS */

:FNTGRPE

MINFID=154

MAXFID=200

FNTSTR='1B5001'X

FNTEND='1B5000'X

FNTWTH=''X.

/* 15 CPI */

:FNTGRPE

MINFID=211

MAXFID=239

FNTSTR='1B570014141B4104'X

FNTEND=''X

FNTWTH=''X.

/* 5 CPI */

:FNTGRPE

MINFID=240

MAXFID=246

FNTSTR='121B5701'X

FNTEND='1B5700'X

FNTWTH=''X.

/* 17.1 CPI */

:FNTGRPE

MINFID=247

MAXFID=259

FNTSTR='1B570014141B4105'X

FNTEND=''X

FNTWTH=''X.

/* 8.57 CPI */

:FNTGRPE

MINFID=260

MAXFID=273

FNTSTR='120F1B5701'X

FNTEND='1B5700'X

FNTWTH=''X.

/* 20 CPI */

:FNTGRPE

MINFID=274

MAXFID=299

FNTSTR='1B570014141B4106'X

FNTEND=''X

FNTWTH=''X.

:EFNTGRP.

/* Individual Fonts */

:INDFNT.

/* OCR B font (10 cpi) */

:INDFNTE

FID=3

POINTSIZE=0

FNTSTR='1B491112'X

FNTEND='1B4900'X

FNTWTH=''X.

/* OCR A font (10 cpi) */

:INDFNTE

FID=19

POINTSIZE=0

FNTSTR='1B491012'X

FNTEND='1B4900'X

FNTWTH=''X.

/* 16.6 CPI (FGID 400) */

:INDFNTE

FID=400

POINTSIZE=0

FNTSTR='1B570014141B4105'X

FNTEND=''X

FNTWTH=''X.

:EINDFNT.

Default print quality

To have AS/400 print jobs generated with the default print quality of *STD to

print at the printers highest speed, find and modify the print quality tags

as shown below.  Note that jobs generated with print quality of *NLQ will

also print and the highest speed (lowest quality).

/* Print Quality */

:PRTQLTY

QLTYTYPE=TEXT

DATA='1B4901'X.

:PRTQLTY

QLTYTYPE=LETTER

DATA='1B4901'X.

Pagesize tags

To have HPT honor the page size values specified in the print job when using

the IBM4247 (dual continuous forms object), the page rotate parameter must be

sent to zero, and the object must contain a PAGSIZE tag whose width and

length match those specified by the job.  The IBM4247 object contains the

following 4 PAGESIZE tags.

Additional forms sizes may be added.  The PAGWTH and PAGLEN values are calcu-

lated by multiplying the width and length in inches by 1440.  The last byte

in the DATA string is then replaced by the hexidecimal value of the number of

lines on the page. Calculate this by multiplying the form length in inches by

6 and converting the result into its hexadecimal value.  For example, a 3"

form would yield:

3 inches x 6 lines/inch = 18 lines=> '12'X

/* Page Sizes (cont forms) */

:PAGSIZXFM.

/* 13.2" X 8.5" */

:PAGSIZE

PAGWTH=19008

PAGLEN=12240

DATA='1B410C1B321B4333'X.

/*  8.0" X 8.5" */

:PAGSIZE

PAGWTH=11520

PAGLEN=12240

DATA='1B410C1B321B4333'X.

/* 13.2" X 11"  */

:PAGSIZE

PAGWTH=19008

PAGLEN=15840

DATA='1B410C1B321B4342'X.

/*  8.0" X 11"  */

:PAGSIZE

PAGWTH=11520

PAGLEN=15840

DATA='1B410C1B321B4342'X.

:EPAGSIZXFM.