Communications Manager/2

- 5622-078 (C) Copyright IBM Corporation 1988, 1993

X.25 SAMPLE APPLICATION

The X.25 sample application consists of programs that demonstrate the

use of the X.25 interface verbs.  The function of the sample application

is to transmit data, using the X.25 protocol, from one PS/2 and receive

that data on the same or a different PS/2.  To support this function,

the sample application consists of two programs.  One is a receive

program, which is referred to as the receiver side.  The other is a

transmit program, which is referred to as the transmitter side.  On the

receiver side, a user starts the receiver program, supplying a routing

table entry name that has been created in the configuration file used to

start Communictions Manager.  On the transmitter side, a user starts the

transmitter program, supplying a directory entry name that has been

created in the configuration file used to start Communictions Manager.

The sample programs are designed to work in the OS/2 protected mode

with Communications Manager.  The sample program source code and the

supporting files are for C and MASM languages.

PREREQUISITES

To run the sample program, a user must be familiar with the following

things:

1.  OS/2 installation and command prompt

2.  TOOLKIT installation

3.  Communications Manager installation

4.  Compiler of the language in which you are interested

COMPILING THE SAMPLE PROGRAMS

1.  Change directory to the directory containing the sample program

(path\X25).

2.  Add the path of the include files for Communications Manager to your

INCLUDE environment variable.

3.  Add the path of the library files for Communications Manager to your

LIB environment variable.

4.  Compile the program using the "nmake" command for your compiler:

nmake x25recv.mak

nmake x25xmit.mak

CREATING AN X.25 CONFIGURATION FILE.

There are 2 possible configurations that can be used to run the X.25

sample programs.

A) Two PS/2's each with an IBM X.25 Interface Co-Processor/2 adapter

installed and connected using ISDN.

B) Two PS/2's each with an IBM X.25 Interface Co-Processor/2 adapter

installed and connected through a modem eliminator.

CONFIGURATION STEPS

The following instructions will configure two OS/2 workstations with

Communications Manager to run the X.25 sample programs.

On the workstation that will run the receiver side of the sample

program, start Communications Manager using X25RECV as the startup

configuration file.

On the workstation that will run the transmitter side of the sample

program, start Communications Manager using X25XMIT as the startup

configuration file.

Create the configuration files for the X.25 transmitter side using

the configuration services in Communications Manager by following

the instructions below:

At command prompt:

Type CMSETUP

Communication Manager Setup:

'CM Setup' panel appears and press Setup...

Open Configuration:

Enter 'X25XMIT' in Configuration field

Enter 'X25 Transmit Sample Configuration' in Description field

Press OK

OS/2 Communications Manager:

Press Yes to create New Configuration File

OS/2 Communications Manager:

Press Yes if you want to use this workstation as the transmitter

side, otherwise press No.

Communications Manager Configuration Definition:

If you are set up for configuration B,

Select 'X.25' and 'X.25 API's' and then press Configure...

Communications Manager Profile List Sheet:

Select 'X.25 links' in Profile Name and then press Configure...

X.25 Links:

Press Create

X.25 Link Parameters:

Enter 'LINK1' in Link name field

Select 'X.25' in Adapter type field

In Slot number field, select the slot number where the IBM

X.25 adapter is located in the transmitting workstation

Select Network type according to the the X.25 network subscription (see help)

Select Local CCITT compliance according to the X.25 network

subscription (see help)

Select 'DTE' in Link setup mode field

In Local DTE address field, enter the address assigned to your DTE

Enter comment if desired in Optional comment field

Select "Virtual circuit ranges' under Additional Parameters

Press Change...

Virtual Circuit Ranges:

Enter the appropriate information in these fields depending on

your X.25 network subscription

Press OK

X.25 Link Parameters:

Press OK

X.25 Links:

Press Close

Communications Manager Profile List Sheet

Select X.25 directory entries

Press Configure...

X.25 Directory Entries:

Press Create...

Create Directory Entry:

Enter 'REMOTE1' in Directory entry name field

Press OK

SNA SVC Directory Entry:

Select 'LINK1' in Link name field

Select 'Primary' in Link station role field

Select the CCITT compliance according to the receiving

workstation's X.25 network subscription (see help)

In Remote DTE address field, enter the address of the

receiving workstation

Enter comment if desired in Optional comment field

Press OK

X.25 Directory Entries:

Press Close as needed to complete the configuration

Create the configuration files for the X.25 receiver side using

the configuration services in Communications Manager by following

the instructions below:

At command prompt:

Type CMSETUP

Communication Manager Setup:

'CM Setup' panel appears and press Setup...

Open Configuration:

Enter 'X25RECV' in Configuration field

Enter 'X25 Receive Sample Configuration' in Description field

Press OK

OS/2 Communications Manager:

Press Yes to create New Configuration File

OS/2 Communications Manager:

Press Yes if you want to use this workstation as the receiver

side, otherwise press No.

Communications Manager Configuration Definition:

If you are set up for configuration B,

Select 'X.25' and 'X.25 API's', then press Configure...

Communications Manager Profile List Sheet:

Select X.25 links in Profile Name and then press Configure...

X.25 Links:

Press Create

X.25 Link Parameters:

Enter 'LINK2' in Link name field

Select 'X.25' in Adapter type field

In Slot number field, select the slot number where the IBM

X.25 adapter is located in the receiving workstation

Select Network type according to the the X.25 network subscription (see help)

Select Local CCITT compliance according to the X.25 network

subscription (see help)

Select 'DTE' in Link setup mode field

In Local DTE address field, enter the address assigned to the DTE

Enter comment if desired in Optional comment field

Select "Virtual circuit ranges' under Additional Parameters

Press Change...

Virtual Circuit Ranges:

Enter the appropriate information in these fields depending on

the X.25 network subscription

Press OK

X.25 Link Parameters:

Press OK

X.25 Links:

Press Close

Communications Manager Profile List Sheet

Select X.25 routing table

Press Configure...

X.25 Routing Table:

Press Create...

Routing Table Entry:

Enter 'ROUTE1' in Directory entry name field

Press OK

X.25 Routing Table:

Press Close as needed to complete the configuration

Refer to the following publications for configuration procedures:

(1)  Configuration Guide (SC31-6171)

(2)  Scenarios (SC31-6172)

(3)  X.25 Programming Reference (Chapter 5).