Script for Module 2, Topic 2

| Module 2, Topic 2 | Event Type and Description: Topic introduction page with embedded background graphic and additional graphic | Screen # 1.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | (1) Background graphic: topic_intro.jpg (2) 30_wiifm.jpg |
| --- | --- |
| Alt Text | (1) N/A (2) What’s in it for you? |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction Physical Components of the Mainframe Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Topic Introduction In this topic, you will have an opportunity to virtually assemble a mainframe, learning about the sections as you build it. Then your guide, Courtney, returns to help you understand from a high-level perspective the key differences in mainframe hardware and how it compares to a distributed system. She will also discuss the major operating systems and software that run on System z. [30_wiifm.jpg] By taking the mystery out of the mainframe and learning what is in the box, you can discuss more confidently the mainframe and its benefits with your customers. You will also continue to learn how the mainframe is an excellent fit for certain customers, based on their business requirements. [30_wiifm.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic and Flash activity | Screen # 2.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | (1) m2_t2_witb.gif (2) Update with new Graphics from Ian - access_a.gif (inactive); access_b.gif (active) |
| --- | --- |
| Alt Text | (1) What’s in the box? The mainframe, demystified. (2) Accessible version button |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Physical Components of the Mainframe To become more familiar with the physical components of the mainframe, click the following image to begin virtually building your mainframe. The activity opens in a separate window; close that window to return to this page. [m2_t2_witb.gif] For an accessible version of the activity, click this button: Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text in pop-up window | Screen # N/A |
| --- | --- | --- |

| Comments | This is the accessible version of the platform challenge activity launched on Screen 2.00. |
| --- | --- |

Event Text:

Accessible Version of the Mainframe Assembly Activity After the front doors of the System z10 EC are opened, the left view consists of the Z frame and its associated parts and the right view consists of the A frame and its associated parts. The Z frame includes these parts, from top to bottom: - Integrated batteries - System power supply - Input/Output (I/O) Cage 3 - Redundant support elements - I/O Cage 2 The A frame includes these parts, from top to bottom: - Integrated batteries - Central Electronic Complex (CEC) - InfiniBand I/O Interconnects - Cooling equipment - I/O Cage 1 - Fiber Quick Connect (FQC) The following information describes these parts. Integrated Batteries The optional Internal Battery Feature (IBF) provides a local uninterrupted power source. The IBF further enhances the robustness of the power design, increasing power line disturbance immunity. It provides battery power to preserve processor data in case of a loss of power on all four AC feeds from the utility company. The IBF can hold power briefly over a brownout or for orderly shutdown, in case of a longer outage. The IBF provides up to 10 minutes of full power, depending on the I/O configuration. System Power Supply The System z family of servers provides high availability by helping to reduce both planned and unplanned outages. The System z N+1 power supply technology helps to achieve reductions in outages. As a side note, a fully-loaded System z10 EC server maximum power consumption is 31.7 kilowatt. I/O Cage 3—Optional I/O features are installed in I/O cages. I/O Cage 1 is located in the bottom of the A Frame. Optional I/O cages are added in the bottom of the Z frame (I/O Cage 2), and the upper position in the Z frame (I/O Cage 3), in that order, as needed. One I/O cage accommodates the following card types: - Up to 8 Crypto Express2 - Up to 28 FICON® Express4, FICON Express2, or FICON Express - Up to 24 OSA-Express2 and OSA-Express3 - Up to 28 ESCON Note: It is possible to populate the 28 I/O slots in one I/O cage with any mix of these cards. Redundant Support Elements A support element is a dedicated workstation used for monitoring and operating a system. It is attached to the central processor complex (CPC) of a system. If you have experience using other systems, you might have used a processor console, support processor, or a similarly named workstation to monitor and operate them. The System z10 EC has an integrated support element, that is, the support element is located inside the same frame in which the CPC is located. An alternate support element is also provided to give the option to switch from the primary support element to the alternate support element if hardware problems occur. Central Electronic Complex (CEC) The CEC is housed in its own cage, and the cage houses from one to four fully interconnected processor books. Each book contains a multichip module (MCM), memory, I/O cage connectors, and, optionally, coupling link connectors. The System z10 EC is built on the proven superscalar microprocessor architecture, which was deployed on the System z9 EC. However, the new processor unit (PU) chip in System z10 has several distinctive innovations—notably in error checking and correcting—and specialized circuitry to support, for example, decimal floating point operations. It has a 4.4 GHz high-speed, quad-core design. One MCM has five PU chips and two storage control (SC) chips in each book. Each PU chip has either three or four cores. The MCM is cooled by modular refrigeration units (MRUs) with air cooling backup. In any model of the server, two cores are designated as spares, and each individual core can be transparently spared, as contrasted with previous systems in which the chip was the sparing unit. In each book, up to 384 GB of memory can be installed. Because 16 GB are part of the base and are reserved for the hardware system area (HSA), the maximum amount of purchasable memory is 1520 GB, just shy of 1.5 TB. The InfiniBand (IFB) specification includes an I/O architecture for interconnecting communications infrastructure equipment and IT equipment, including storage, servers, and embedded systems. The architecture uses point-to-point channels in a switched connection, thus constituting a fabric. Connections can be either external, using both copper cable and optical fiber, or internal, as in chassis backplanes. With IFB, very high data transfer speeds are achievable, and a single connection is capable of carrying several types of traffic, such as communications, management, clustering, and storage. Additional characteristics include low processing overhead, low latency, and high bandwidth. The IFB fanouts are located in the front of each book; each book has up to eight fanout slots. Each fanout has two ports to connect an Integrated Cluster Bust (ICB) or IFB cable, depending on the type of fanout. There are two types of Host Channel Adapters (HCA): - The HCA2-C is a copper cable that connects to an I/O cage. - The HCA2-O provides optical connections for InfiniBand I/O Interconnects. InfiniBand I/O Interconnects In the System z10 EC server, the cables from the CEC cage to the I/O cages carry the InfiniBand (IFB) protocol. For external usage, Parallel Sysplex InfiniBand (PSIFB) links are introduced to keep up with server speed. They are used to interconnect System z servers in a Parallel Sysplex. Cooling Equipment The System z10 EC is an air-cooled system assisted by refrigeration. Refrigeration is provided by a closed-loop liquid cooling subsystem. The entire cooling subsystem has a modular construction. In addition to the refrigeration unit, an air-cooling backup system is in place. Cooling components and functions, found throughout the cages, are made up of two subsystems: the modular refrigeration unit (MRU) and the motor drive assembly (MDA). One or two MRUs (MRU0 and MRU1), located in the A Frame below the books, provide refrigeration to the content of the books together with two large blower assemblies: motor scroll assembly (MSA) and MDA at the rear of the CEC cage (one for each MRU). Each of these assemblies is connected to the bulk power adapter (BPA), which regulates cooling by increasing the blower speed in combination with an air moving assembly in the top part of the CEC cage. MDAs are found throughout the frames to provide air cooling where required. They are located at the bottom front of each cage, and in between the CEC cage and I/O cage, in combination with the MSAs. Fiber Quick Connect (FQC)—Optional The optional FQC feature enables trunk cables to connect to Fiber Connection (FICON) channels or Enterprise Systems Connection (ESCON) channels using under-the-cover attachment harnesses. These harnesses are installed when system is built, and the System z10 EC arrives ready to connect the trunk cables at the customer site. FQC is designed to significantly reduce the amount of time required for on-site installation and setup of fiber optic cabling. FQC facilitates additions, moves, and changes of ESCON and FICON LX fiber optic cables in the data center and could reduce fiber connection time by up to 80%. FQC is for factory installation of IBM Facilities Cabling Services—Fiber Transport System (FTS) fiber harnesses for connection to channels in the I/O cage. FTS fiber harnesses enable connection to FTS direct-attach fiber trunk cables from IBM Global Technology Services. FQC supports all of the ESCON channels and all of the FICON LX channels in all of the I/O cages of the server.

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 3.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_scene9.jpg |
| --- | --- |
| Alt Text | Photograph of your guide, Courtney |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe Now that you are more familiar with the physical components of the mainframe, your guide, Courtney, has returned to help you understand: [m2_t2_scene9.jpg] - Key hardware differences of the mainframe - Operating systems that run on System z - System z software offerings Courtney takes the discussion from here. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 4.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_scene10.jpg |
| --- | --- |
| Alt Text | Photograph of your guide, Courtney |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) [m2_t2_scene10.jpg] Hello again! Let’s talk about some of the key hardware differences of the mainframe, specifically system assist processors and multiple engines. First, let’s focus on the processors in System z and how they compare with distributed systems. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 5.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_processors.jpg |
| --- | --- |
| Alt Text | This image depicts the mainframe environment and the distributed system environment. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) General Purpose Processors versus Central Processors A distributed system has one or more general purpose processors, like an Intel Pentium or a Power 6 processor. When I say it’s a general purpose processor, I mean that this processor runs the applications and all of their associated work. If an application wants to add two plus two, get data from a disk drive, or send a message across a network, it uses this general purpose processor. There is a slightly different term for this with System z. It’s called a central processor, and you’ll see why in a minute. If that same application wants to add two plus two, the System z central processor does the math. But getting data from a disk drive or sending a message across a network? The central processor doesn’t do these tasks. Why not? The central processor is, in system terms, an expensive and scarce resource that can be shared by a lot of different work. Having it wait around for those slow disk drives and networks is not very productive. Remember that System z runs multiple applications, and distributed systems typically run only one. So, distributed systems can afford to wait; there is no other productive work to be done. With System z, the central processors can be freed up to do valuable work for other applications. Click next to continue. [m2_t2_processors.jpg] |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 6.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_sap.jpg |
| --- | --- |
| Alt Text | This image represents system assist processors. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) System Assist Processors How does the mainframe’s central processor get freed up? System z includes specialized processors called system assist processors that handle the “slow” stuff. If you had a typical 4-way (four general purpose processors) distributed server from Dell or HP, and you had a 4-way System z mainframe, you would actually have more processors in the mainframe. You would have system assist processors that handled input and output (I/O), things like data encryption, systems management tasks, and error recovery. [m2_t2_sap.jpg] So, does this all really make a difference? You bet. Recently, a large financial services institution wanted to see what kind of a difference it would make if they moved a large Oracle database from their Sun servers to a System z mainframe. It was a huge database (36 trillion bytes of data), and they had a large daily batch job and an even larger weekly batch job to run. The daily batch job, when running on the Sun, took 15 minutes. When it ran on the mainframe, it finished in four minutes. The weekly batch job? Five hours and 50 minutes on the Sun and only 45 minutes on the mainframe. Yes. It makes a big difference. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 7.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Match the term on the left with its description on the right. A. General purpose processor; - This processor handles “slow” input and output (I/O) work, such as data encryption, systems management tasks, and error recovery.. B. Central processor; - This processor runs system applications and all of their associated work.. C. System assist processor; - This processor is, in system terms, an expensive and scarce resource.. Correct answer: A – 2 B – 3 C – 1 Correct answer feedback: Nice work! Incorrect answer for first and second attempts: Good try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good attempt, but the correct answer is A – 2, B – 3, and C – 1. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 8.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_psys1.jpg |
| --- | --- |
| Alt Text | This image represents the ability of System z to tie together two or more physically different systems so that they appear as one system. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) Parallel Sysplex Just how reliable is that mainframe? Extremely. When talking about the reliability of system hardware, we measure the “mean time between failures,” or MTBF. This is how long, on average, it takes a piece of hardware to fail. Most distributed systems hardware, depending on how much effort went into its manufacture, measure the MTBF in years—and that is the high-end hardware. The mainframe hardware MTBF is measured in decades. It very rarely fails. But, what if the mainframe does fail? System z mainframes provide the ability to take two or more physically different systems and tie them together so that they look like one system. The individual systems become nodes in what is called a Parallel Sysplex. “Sysplex” is an abbreviation that stands for “systems complex.” The individual systems are called members in a Parallel Sysplex. If one member of the Parallel Sysplex fails, the remaining members assume the workload and keep everything running, in most cases without the end user seeing the failure. As far as the end user is concerned, there was no failure. [m2_t2_psys1.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 9.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_psys2.jpg |
| --- | --- |
| Alt Text | This image represents clustering in the distributed system environment. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) Parallel Sysplex (continued) [m2_t2_psys2.jpg] The distributed world tries to do the same thing. They call lashing multiple systems together “clustering,” and the individual systems become nodes in a cluster. There are two kinds of clusters: high-performance clusters (mostly used for scientific types of workloads) and high-availability clusters (mostly used to keep business applications up and running). Our focus in this topic is high-availability clusters. The distributed high-availability clusters can run in two modes: active/passive clusters and active/active clusters. In an active/passive cluster, one node is processing the work and another node is sitting idle, waiting to kick in if there is a failure. In an active/active cluster, both nodes are used to run business applications. The active/active cluster approach is the closest to the System z Parallel Sysplex, but the differences in functionality could be considered major. First, without the Workload Manager, the distributed systems have to rely on those not-so-intelligent workload distributors that we talked about previously. System z can distribute the workload where the available resources can best handle it. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 10.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_psys3.jpg |
| --- | --- |
| Alt Text | This image represents how, in a clustered distributed environment, one system sends a “heartbeat” to the other systems and waits for their responses. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) Parallel Sysplex (continued) Let’s talk about another difference in the hardware. The System z Parallel Sysplex includes a component that has the specific purpose of keeping track of activity within the systems. So, information that is of a global nature (of interest to all the systems) is kept in this component. That way, if a system within the Parallel Sysplex fails, other systems can pick up where the failing system left off. The distributed systems? Clusters use a strategy called, “Guess what’s going on in the other system?” The most popular technique is to guess when another system has failed and then try to recover. One system sends the other systems a “heartbeat,” and if it doesn’t detect the heartbeat, it goes into recovery. The problem is…was the system just too busy to send a heartbeat? Did some temporary problem cause it to delay sending the heartbeat, but now everything is fine? Sorry…too late. A very costly—and now probably disruptive—takeover is in progress. That’s not to say that active/active clustering doesn’t work. If you throw a whole lot of people at it, it works…sometimes. [m2_t2_psys3.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 11.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_psys4.jpg |
| --- | --- |
| Alt Text | This image represents that Parallel Sysplex technology has the ability to perform hardware and software maintenance and installations in a nondisruptive manner. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) Parallel Sysplex (continued) If anybody asks you, the secret ingredient that enables the Parallel Sysplex to provide high performance along with high availability is the coupling facility. The coupling facility enables all the members in a Parallel Sysplex to know what the other members are doing—with a minimum of overhead. [m2_t2_psys4.jpg] Another significant and unique advantage of using Parallel Sysplex technology is the ability to perform hardware and software maintenance and installations in a nondisruptive manner. Through data sharing and dynamic workload management, servers can be dynamically removed from or added to the cluster. This allows installation and maintenance activities to be performed while the remaining systems continue to process work. Furthermore, by adhering to IBM's software and hardware coexistence policy, software and hardware upgrades can be introduced one system at a time. This capability allows customers to roll changes through systems at a pace that makes sense for their business. The ability to perform rolling hardware and software maintenance in a nondisruptive manner allows businesses to implement critical business functions and to react to rapid growth without affecting customer availability. If you want more information, be sure to visit Parallel Sysplex. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 12.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_scene11.jpg |
| --- | --- |
| Alt Text | Photograph of your guide, Courtney |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Hardware Differences of the Mainframe (continued) Now it’s time for a few checkpoint questions. After that, we’ll shift our focus to the operating systems that run on System z. [m2_t2_scene11.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 13.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Which one of the following terms is used when referring to the reliability of system hardware? A. Mean time between breakdowns B. Mean time between flareups C. Mean time between fixes D. Mean time between failures Correct answer: D Correct answer feedback: Nice job! Mean time between failures (MTBF) is used when referring to the reliability of system hardware. Incorrect answer for first and second attempts: Good attempt, but this answer is incorrect. Incorrect answer feedback for third attempt: Good try, but the correct answer is D. Mean time between failures (MTBF) is used when referring to the reliability of system hardware. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 14.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question True or false? System z mainframes provide the ability to take two or more physically different systems and tie them together so that they look like one system; the individual systems become nodes in what is termed a Parallel Complex. Correct answer: False Correct answer feedback: Good job! The individual systems become nodes in what is termed a Parallel Sysplex. Incorrect answer: True Incorrect answer feedback: Good try, but this statement is false. The individual systems become nodes in what is termed a Parallel Sysplex. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 15.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question In a distributed system environment, what term is used to describe tying multiple systems together so that they appear as one system? A. Noding B. Clustering C. Lashing D. Joining Correct answer: B Correct answer feedback: Nice work! In a distributed system environment, “clustering” is the term used to describe tying multiple systems together so that they appear as one system. Incorrect answer for first and second attempts: Good try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good attempt, but the correct answer is B. In a distributed system environment, “clustering” is the term used to describe tying multiple systems together so that they appear as one system. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 16.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Which one of the following types of distributed system clusters is most similar to the System z Parallel Sysplex in terms of functionality? A. Active cluster B. Passive cluster C. Active/active cluster D. Active/passive cluster Correct answer: C Correct answer feedback: Good job! The distributed system active/active cluster—in which both nodes are used to run business applications—is most similar to the System z Parallel Sysplex in terms of functionality. Incorrect answer for first and second attempts: Nice try, but this answer is incorrect. Incorrect answer feedback for third attempt: Nice attempt, but the correct answer is C. The distributed system active/active cluster—in which both nodes are used to run business applications—is most similar to the System z Parallel Sysplex in terms of functionality. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 17.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Which component in System z enables the Parallel Sysplex to provide high performance along with high availability? A. Coupling device B. Coupling node C. Coupling resource D. Coupling facility Correct answer: D Correct answer feedback. Good work! The coupling facility enables the Parallel Sysplex to provide high performance along with high availability. Incorrect answer for first and second attempts: Good try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good attempt, but the correct answer is D. The coupling facility enables the Parallel Sysplex to provide high performance along with high availability. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 18.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question True or false? Parallel Sysplex technology allows installation and maintenance activities to be performed while the remaining systems continue to process work. Correct answer: True Correct answer feedback: Nice job! Parallel Sysplex technology allows installation and maintenance activities to be performed while the remaining systems continue to process work. Incorrect answer: False Incorrect answer feedback: Good attempt, but this statement is true. Parallel Sysplex technology allows installation and maintenance activities to be performed while the remaining systems continue to process work. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 19.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_zos1.jpg |
| --- | --- |
| Alt Text | This image highlights two System z operating systems, z/OS and z/VM. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Operating Systems The System z hardware platform can run a number of different operating systems, and those operating systems can run a lot of the most important IBM middleware offerings. The premier operating system, and the one we’ve been discussing in this module, is the System z Operating System, or z/OS. z/OS has been around in one form or another since the 1960s, and applications developed back then can still run on the latest version of z/OS today. This has provided incredible value to System z customers because they can continue to leverage software development investments made 40 years ago. z/OS provides the highest Qualities of Service (QoS)—reliability, availability, security, and performance—of any commercial operating system available today. Your customers might make use of other System z operating system offerings. The z Virtual Machine, or z/VM, offering provides a powerful virtualization environment, as well as a first-class environment for decision support systems. z/VM has become very popular these days as a provisioning tool for large, enterprise-class deployments of Linux on IBM System z. [m2_t2_zos1] Linux on System z enables customers to run the Linux OS on System z hardware, along with a huge number of middleware and application offerings targeted for the Linux environment. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 20.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_zos2.gif |
| --- | --- |
| Alt Text | This image explains when to deploy z/OS and Linux on System z. Technical considerations include: - Qualities of Service (QoS), in which z/OS is the preferred operating system - Speed of deployment, in which Linux on System z is the preferred operating system - Degree of portability, in which Linux on System z is the preferred operating system Other considerations when determining when to deploy z/OS and Linux on System z include: - Application availability - Workload management function and granularity - File sharing across a sysplex - Manageability and scaling - Availability of skill |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Operating Systems (continued) [m2_t2_zos2.gif] If you’re talking to a new customer about System z, chances are you’ll be discussing z/OS or Linux on System z. How do you decide which one to recommend? There are some basic requirements that will help you make your decision. Take a look at these comparison considerations: Let’s summarize how you would position these two operating system choices. - z/OS provides the: - Highest QoS (reliability, availability, security, and performance) - Highest integration among applications - Highest utilization of the hardware - Linux on System z provides the: - Best virtualization - Highest degree of portability - Significant cost savings in appropriate situations - Next best way to integrate open systems with z/OS information Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 21.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_zos3.jpg |
| --- | --- |
| Alt Text | This image highlights two other System z operating systems, z/VSE and z/TPF. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Operating Systems (continued) I mentioned previously that z/OS had been around since the 1960s. Also around since the ‘60s are two other operating systems: z Virtual Storage Extended, or z/VSE, and z Transaction Processing Facility, or z/TPF. z/VSE was a very popular operating system for customers with smaller computing requirements. z/VSE provided an application platform that required fewer skills to administer than did z/OS. There are many shops today that still are devoted to z/VSE. z/VSE has had some connectivity enhancements in the past few years that enable customers to share information with their distributed environments. z/TPF provided a high-performance, highly efficient environment for transactional processing. It required highly skilled programmers to configure it properly, but the payoff was high levels of performance and availability that no other general purpose computer system could offer. z/TPF remains popular with the airlines and some financial institutions, but it is also a viable option for any customer or group of customers whose business model focuses on investing in an operating environment that can process extreme volumes of transactions at the lowest possible cost per transaction. The latest version of z/TPF embraces open systems coding and service-oriented architecture, or SOA, technologies. For an introduction to z/TPF, listen to the interview at: “Did you say Mainframe?!”: An introduction to z/TPF. [m2_t2_zos3.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 22.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_scene12.jpg |
| --- | --- |
| Alt Text | Photograph of your guide, Courtney |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Operating Systems (continued) That’s a high-level overview of the operating systems that run on System z. The middleware that runs on System z includes all the functions you would normally associate with high-volume, high-function application systems. After a few checkpoint questions, we’ll talk about those software offerings. [m2_t2_scene12.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 23.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Which of the following operating systems provided a high-performance, highly efficient environment for transactional processing at its inception in the 1960s? A. z/TPF B. z/OS C. z/VSE D. Linux on System z Correct answer: A Correct answer feedback. Good job! In the 1960s, z/TPF provided a high-performance, highly efficient environment for transactional processing; it remains popular with airlines and some financial institutions today. Incorrect answer for first and second attempts: Good try, but this answer is incorrect. Incorrect answer feedback for third attempt: Nice attempt, but the correct answer is A. In the 1960s, z/TPF provided a high-performance, highly efficient environment for transactional processing; it remains popular with airlines and some financial institutions today. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 24.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Your customer’s key business requirements include the highest reliability, availability, security, and performance available. Which one of the following would you include in your discussion with your customer? A. z/OS B. z/TPF C. z/VSE D. Linux on System z Correct answer: A Correct answer feedback. Good job! For customers who require the highest QoS (reliability, availability, security, and performance), z/OS should be included in the discussion. Incorrect answer for first and second attempts: Good try, but this answer is incorrect. Incorrect answer feedback for third attempt: Nice attempt, but the correct answer is A. For customers who need the highest QoS (reliability, availability, security, and performance), z/OS should be included in the discussion. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 25.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Your customer’s key business requirements include the highest integration among applications. Which one of the following would you include in your discussion with your customer? A. z/OS B. z/TPF C. z/VSE D. Linux on System z Correct answer: A Correct answer feedback. Nice work! For customers who require the highest integration among applications, z/OS should be included in the discussion. Incorrect answer for first and second attempts: Good attempt, but this answer is incorrect. Incorrect answer feedback for third attempt: Nice try, but the correct answer is A. For customers who require the highest integration among applications, z/OS should be included in the discussion. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 26.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Your customer’s key business requirements include the highest utilization of the hardware. Which one of the following would you include in your discussion with your customer? A. z/OS B. z/TPF C. z/VSE D. Linux on System z Correct answer: A Correct answer feedback. Nice job! For customers who require the highest utilization of the hardware, z/OS should be included in the discussion. Incorrect answer for first and second attempts: Nice try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good attempt, but the correct answer is A. For customers who require the highest utilization of the hardware, z/OS should be included in the discussion. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 27.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Your customer’s key business requirements include the best virtualization available. Which one of the following would you include in your discussion with your customer? A. z/OS B. z/TPF C. z/VSE D. Linux on System z Correct answer: D Correct answer feedback. Good job! For customers who require the best virtualization available, Linux on System z should be included in the discussion. Incorrect answer for first and second attempts: Nice try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good try, but the correct answer is D. For customers who require the best virtualization available, Linux on System z should be included in the discussion. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 28.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_software1.jpg |
| --- | --- |
| Alt Text | This image represents System z software offerings. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Software Offerings Customer Information Control System, or CICS®, is a transaction processing environment that allows applications to share information and to satisfy customer requirements for information. Think of CICS as one of the first application servers. CICS application programs could be written in Common Business-Oriented Language (COBOL), Programming Language One (PL/1), or Assembler. Most of your customers used COBOL, but there are still quite a few viable systems written in the other two languages. CICS also added Java as one of its supported languages. CICS runs on the z/OS and z/VSE operating systems. [m2_t2_software1.jpg] Information Management System Transaction Manager, or IMS TM, is like CICS in that it also provides a transaction processing environment. Also an application server, it enables application programs written in COBOL, PL/1, or Assembler to run in a shared services environment. IMS has also added Java as a fully supported language environment. IMS runs only on z/OS. Information Management System Database Server, or IMS DB, is a database system that is capable of extremely high performance. While it lacks the functionality of a full relational database server, it is ideal for storing operational data in a very efficient manner. It is also built for high-speed access. IMS DB runs on z/OS, and the databases can be accessed by either IMS TM or CICS transaction programs. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphic | Screen # 29.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | m2_t2_software2.jpg |
| --- | --- |
| Alt Text | This image represents System z software offerings. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Software Offerings (continued) [m2_t2_software2.jpg] The DB2 that your customers know and love started life on z/OS. DB2 provides a full, functional relational database that leverages all of the features of System z and z/OS to provide high availability and high performance. DB2 also runs on all the other System z operating systems. And that application server that your customer uses? WebSphere Application Server (WAS)? Yes, it runs on z/OS, and it provides true application portability between System z and distributed systems. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with table | Screen # 30.00 |
| --- | --- | --- |

| Table Summary | This table compares the distributed system, System z with z/OS, and System z with Linux in the following areas: languages, application servers, databases, communications (TCPIP), and operating systems. |
| --- | --- |

| Comments | Shading in table cells should be consistent with Module 1 tables. |
| --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Software Offerings (continued) If you think of the traditional application “stack,” the following table describes how the System z middleware fits when compared to a distributed system. Distributed; System z with z/OS; System z with Linux. Languages; C++ Java; COBOL PL/1 Java C; C++ Java. Application Servers; WAS; WAS CICS IMS TM; WAS. Databases; DB/2 Oracle; DB/2 IMS DB; DB/2. Communications (TCPIP); Built in; z/OS Comm Server; Built in. Operating Systems; AIX Linux Windows; z/OS; Linux on System z. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Checkpoint question | Screen # 31.00 |
| --- | --- | --- |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | Checkpoint Question Which of the following provides a full, functional relational database that enables high availability and high performance? A – IMS TM B – DB2 C – WAS D – CICS Correct answer: B Correct answer feedback. Good job! DB2 provides a full, functional relational database that leverages all of the features of System z and z/OS to provide high availability and high performance. It runs on all System z operating systems. Incorrect answer for first and second attempts: Nice try, but this answer is incorrect. Incorrect answer feedback for third attempt: Good try, but the correct answer is B. DB2 provides a full, functional relational database that leverages all of the features of System z and z/OS to provide high availability and high performance. It runs on all System z operating systems. Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Text with graphics | Screen # 32.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | (1) m2_t2_scene13.jpg (2) m2_t2_summary.jpg |
| --- | --- |
| Alt Text | (1) Photograph of your guide, Courtney (2) This image represents the inherent strengths of System z. |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems System z Software Offerings Topic Summary | System z Software Offerings (continued) [m2_t2_scene13.jpg] You might think that I’m telling you that you should always sell System z to run your customers’ applications. However, that’s not the case. What I hope you take away from our discussion is an understanding of the inherent strengths—the “sweet spots”—of the System z mainframe. By understanding the System z sweet spots, you increase your value to the customer as a seller and as a consultant. If you ask the customer what their critical success factors are for the deployment of their new application, and you hear… - High availability - High security - Massive workload requirements - Risky or undefined workload requirements (for example, “If this takes off, it will be huge!”) - The need for tight integration with existing mainframe-based systems …then you should lead with System z. These are the System z sweet spots, and you will best serve your customer and IBM by proposing System z. [m2_t2_summary.jpg] Click next to continue. |
| --- | --- |

| Module 2, Topic 2 | Event Type and Description: Topic summary page with embedded background graphic | Screen # 33.00 |
| --- | --- | --- |

Event Graphic(s):

| Graphic Name | (1) Background graphic: summary.jpg (2) 32_feedback.gif |
| --- | --- |
| Alt Text | (1) N/A (2) N/A |

Event Text:

| - Module Menu –Module Overview and Objectives Topic 1: What Does a Server Do, and What Does an Ideal Server Do?  Topic 2: Why the Mainframe Is Just Another Computer, but a Different Kind of Computer Topic Introduction  Physical Components of the Mainframe  Hardware Differences of the Mainframe  System z Operating Systems  System z Software Offerings  Topic Summary | Topic Summary In this topic, you learned: - How the physical components of the mainframe are arranged and what their functions are - What some of the key hardware differences of the mainframe are and how they help provide for QoS - What the major software operating systems and the major software offerings are that run on System z - What the advantages and differences are between Linux on System z and z/OS Continue to Module 3: The Future Runs on System z to learn how System z: - Is used as a data server, an application server, an application development environment, and a management hub - Inherently has all of the desired future capabilities and QoS that can solve today’s business problems Return to the course launch page at Learning@IBM to download your next module. Please provide your feedback to help us enhance this course, or direct any course-related questions to: Scott F. Harris IBM Certified Project Manager, SWG Global Technical Sales, System z SW 877-817-1898/364-0348 sfharris@us.ibm.com [Insert 32_feedback.gif] |
| --- | --- |