IBM COMMUNICATIONS MANAGER/2 FOR OS/2

API SAMPLE PROGRAMS SUPPORT

(c) Copyright International Business Machines Corp., 1991, 1992

LUA SAMPLE PROGRAM

The LUA sample programs provide examples for writing application

programs using the LUA interface verbs.  The function of the sample

programs is to communicate with a HOST using the LUA interface verbs to

SEND and RECEIVE data, SNA commands, and responses.  The sample scenario

consists of an OS/2 program (secondary), communicating with a HOST

program (primary).

NOTE: There is information included in the source module

header of VTAMPGMA.JCL, the HOST (primary) sample

program. You need to configure the  APPL  table

and  MODE  table entries before execution.

After compiling and linking, a user starts the secondary program,

supplying a filename and path at the OS/2 command prompt.  The primary

sample program is started by submitting the assemble, link, and go job

to MVS.

The secondary sample programs are designed to run only in protected mode

of OS/2.  There are 2 secondary C language sample programs: one using

the RUI interface and one using the SLI interface.  The primary sample

program source code language is 370 ASSEMBLER.

PREREQUISITES

To run the sample programs, a user must obtain and be familiar

with the following:

1.  OS/2 installation and program execution

2.  TOOLKIT installation

3.  Communications Manager installation

4.  Compiler of the language(s) selected

5.  Host VTAM and NCP configuration

HARDWARE REQUIREMENTS

For this example configuration, ensure you have the following hardware.

(a) a PS/2 containing a Multifunction Protocol Adapter, or equivalent

hardware (ie.  PC-AT with SDLC adapter, etc.).

(b) either 2 modems or a modem eliminator with proper cabling.

(c) a Host connection with a 3725 Communication Control Unit or

equivalent communication equipment.

If your hardware configuration is different, then you must select the

appropriate parameters on the Data Link Control Profiles and Partner

Logical Unit Profiles.

Other Data Link types may be substituted where supported.

INSTALLATION

The installation for running LUA sample programs consists of the

following:

1. Installing IBM Communications Manager/2.

2. Creating the LUA configuration file for the sample programs.

3. Installing TOOLKIT

4. Installing the compiler selected

5. Host VTAM and NCP configuration

CONFIGURATION STEPS FOR LUA TO USE SDLC DLC

Create a sample configuration file for the LUA sample program using

the configuration services in Communications Manager by following

the instructions below:

At command prompt:

Type CMSETUP

Communication Manager Setup:

'CM Setup' panel appears and press Setup...

Open Configuration:

Enter 'ACSLUA' in Configuration field

Enter 'LUA Sample Configuration' in Description field

Press OK

OS/2 Communications Manager:

Press Yes to create New Configuration File

OS/2 Communications Manager:

Press Yes to use it for your PC

Communications Manager Configuration Definition:

Select 'SDLC' and 'LUA API's', then press Configure...

Communications Manager Profile List Sheet:

Select DLC-SDLC in Profile Name and press Configure...

SDLC DLC Adapter Parameters:

Select '0' in Adapter field

Select 'Non-switched' in Line type field

Select 'Secondary' in Link station role

Enter '1024' in Maximum I-field size field

Uncheck 'Free unused links' box

Select 'Link station role parameters' under Additional parameters list

Press Change

Link Station Role Parameters:

Enter 'C1' in Local station address (hex)

Press OK

SDLC DLC Adapter Parameters:

Select 'Modem parameters' under Additional parameters list

Press Change

Modem Parameters:

Select 'Half-duplex' in Line mode field

Select 'Full Speed' in Modem speed field

Check 'Use NRZI data encoding' box

Press OK

SDLC DLC Adapter Parameters:

Select 'Window count parameters' under Additional parameters list

Press Change

Window count parameters:

Enter '7' in Send window count field

Enter '7' in Receive window count field

Press OK

SDLC DLC Adapter Parameters:

Press OK

Communications Manager Profile List Sheet:

Press Close

Communications Manager Configuration Definition:

Press Close as needed to complete the configuration

Refer to the following publications for additional configuration

information:

(1) Configuration Guide (SC31-6171)

(2) Scenarios (SC31-6172)

HOST VTAM AND NCP CONFIGURATION

a. Have an assembler available

b. VTAM and NCP GEN to configure the line, PU, and LU for LUA.

c. Modify the VTAM APPL table to include VTAMPGM.

d. Modify the VTAM MODE table to include LUA768RU MODEENT.

COMPILING THE SAMPLE PROGRAMS

1.  Change the directory to the directory containgin the sample program

(path\LUA).

2.  Add the path of the include files for Communications Manager to

your INCLUDE environment variable.

3.  Add the path of the library files for Communications Manager to

your LIB environment variable.

4.  Compile the programs using the "nmake" command for your compiler:

nmake luarui.mak

nmake luasli.mak

RUNNING THE SAMPLE PROGRAMS

Create a new directory SAMPLE on the secondary side workstation.

Copy the secondary sample program (LUARUI.EXE or LUASLI.EXE)

to the SAMPLE directory.

On the primary side, submit the assemble, link, and go, using

VTAMPGMA.JCL, upload it to the HOST with the BINARY option.

Before submitting the VTAMPGMA program, the question marks in

the USER and PASSWORD fields on the JOB statement should be

replaced by your logon ID and password.

It is recommended that the primary sample program be started

prior to the secondary, in that the primary may take several

minutes to complete the assemble, link, and go steps.

Start Communication Manager on the secondary using the

configuration file ACSLUA.

From an OS/2 command prompt, start your secondary program (LUARUI.EXE,

or LUASLI.EXE) with the filename of the SAMPLE program you want to run.