Release Notes for Fax for Domino R4

Welcome to Fax for Domino R4 (FxD). This document provides information about the following:

·	System requirements

·	DFS migration considerations

·	Installation considerations

·	Known problems and limitations

System Requirements

Before installing FxD, make sure your system meets the following requirements:

·	It is a Pentium 166MHz machine or higher.

·	Microsoft NT 4.0 workstation or server software is installed using the most recent service packs.

·	Lotus Domino Server 4.6.3a or higher 4.6.x version is installed.

·	At least 80MB of memory is available; 96MB is preferred.

·	At least 150MB hard disk space is available.

·	One or more analog or digital phone lines is set up.

·	One of the following fax devices is installed or will be installed:

Brooktrout			GammaLink

TR114+I2B		CP4/LSi

TR114+I2C		CPD

TR114+I2D		CPD/220

TR114+I2L		CPi

TR114+I4B		CPi/100

TR114+I4C		CPi/200

TR114+I4D		CPi/200 BRI

TR114+I4L		CPi/400 PCI

TR114+I4P

TR114+I8P

TR114+I8V

TR114+I8V-T1

TR114+I12V

TR114+P2L

TR114+P4L

TruFax 200

DFS Migration Considerations

1.  	Copy your DFS database files to your local Notes/Data directory before installing FxD

If you browse over the network to find your DFS files during install, you may get an error message stating "Error getting server name."  If this happens, you will need to exit install and copy the DFS database files to your local Notes/Data directory.

2.  	DFS migration does not automatically copy device dll or update notes.ini if a device type was not selected during install

You need to select a Brooktrout or GammaLink device during install and complete the device install.

3.	DFS agents are not migrated to FxD

If you created your own agents in DFS, you will have to copy them to FxD. Some fields found in DFS are not in FxD, so you will also need to test and debug.

4.	DFS custom cover pages

If you have a custom cover page form in DFS, the form will migrate to FxD but a sample document will not be created.

5.	DFS GammaLink CPD/220

DFS customers who use the GammaLink CPD/220 with a base I/O address other than the default, 0x350, will need to delete the existing device profile and create a new GammaLink CDP/220 board profile from scratch after install is complete.

6.	DFS GammaLink CPD

DFS customers who use the GammaLink CPD device should check the IO address on the FxD device form after install to make it was correctly migrated.

7.	Check device profiles

After migration is complete, compare the new device profile with any DFS device profiles to make sure all settings have migrated correctly.

FxD Installation Considerations

1. 	FxD beta users may get an error in BTDLLD.DLL when "Loading Fax"

FxD beta customers may get an error with BTDLLD.dll after they upgrade. If you encounter this error, check the system for multiple versions of BTDLLD.dll. The "new" version of the file should be located in the WinNT\System32 directory with a 9/10/98 date and size of 223 KB. Delete the old file located in the Notes or Notes\Data directory. Once the file is deleted, the error should no longer occur.

2. 	Uninstall existing Brooktrout or GammaLink drivers,  services, or toolkits

Uninstall existing Brooktrout or GammaLink software prior to installing FxD. Multiple versions of device software will cause conflicts.

3.	Do not install both Brooktrout and GammaLink devices

You can not install both Brooktrout and GammaLink devices.  The notes.ini settings for one will overwrite the other.

4.	Reboot system after completing device install

FxD will not initialize devices until you reboot.

5.	Load balancing

The FxD Separate Admin database should be replicated to all parent and child servers.

FXD CONFIGURATION CONSIDERATIONS

1.	FxD does not support the DID "Immediate Start" setting

Do not use the "Immediate Start" setting included on the FxD DID device forms.  This setting is not supported in this release.

2.  The device profile for the GammaLink CPi/200 BRI has a number of fields which have restrictions

Field label		Contains			Comments

"Digit timeout in"		The timeout for individual incoming	This field is ignored by FxD

routing digits

"All digits timeout in"	The timeout for all incoming routing	This field is ignored by FxD

digits

"Number of routing digits"	The number of digital routing digits to	The pop-up help incorrectly

be used for incoming faxes		incorrectly indicates that the

termination digit can be used

"DTMF termination digits"	The key to terminate DTMF numbers.	This field must be left empty

"Dial Mode" (in channel	Selects tone or pulse dialing for loop	This field is ignored by FxD

settings)			 lines

"Enable Receive"		Prevents answering a call on the	Recommend leaving it as "Yes"

channel				since an incoming call could be

directed to either channel.  If it is desired

to limit incoming calls, then this should be

done through configuration of the line.

"Enable DTMF detection"	Enables DTMF detection for loop lines.	This field is ignored by FxD

"Dial Wait" (in the		The extra time to wait before dialing.	This field is ignored by FxD

advanced options)

"Dial timeout"		The total wait for the remote fax to	This field is ignored by FxD

answer

"Signal tone duration"	The duration of signal tones while	This field is ignored by FxD

dialing

"Answer on ring"		The ring number on which to answer.	This field is ignored by FxD.

In addition to the above limitations, a local address cannot be provided to restrict incoming calls to that number.

3.  GammaLink CPi/200 BRI

Currently, the TEI (Terminal Endpoint Identifier -- required for ISDN connections) may have to be specified manually (the default value of "0" is for point-to-point lines).   If the identifier is set incorrectly, both transmit and receive will be inoperative.  GammaLink errors number 3935 and 3936 will result when attempting to transmit.

If  the ISDN line is point-to-multipoint, copy the file net3.64 over the file net3.bin in the Notes\FAX\GAMMA directory.

If your line is point-to-point, and the supplier has specified a TEI other than "0",  set the modemctrl 1200 parameter to the desired value using the GammaLink configuration utility.   If the GammaLink settings do not yet exist, the AutoDetect operation needs to be used to set the defaults.   Repeat this for channel 2.

4.	The number of DNIS routing digits on a T1 line must match the number of digits configured by the phone company.

The number of DNIS routing digits specified in the "Inbound Routing Options" section of the device form must match the number configured by the phone company.  Failure to do this may result in mis-routed faxes.

5.	How to determine the I/O address of a PCI board for the device configuration

GammaLink

GammaLink PCI boards (CPi 400 PCI) have a rotary switch on them.  Put the switch position (0 to 15) in the "Board address" field of the device profile.

Brooktrout

Brooktrout PCI boards (TR114+P2L, TR114+P4L) do not have switch settings.  The administrator must discover what address was assigned by the system as follows:

* Configure FxD normally.

* In the server profile, ensure that minor events are being logged.

Create device profiles for all installed boards, but set the PCI board addresses to 4096.  Other boards must be set correctly (according to switch settings).

* Run FxD.

The log will display the address of all boards on the system plus errors for the unfound PCI devices.

Tell FxD to quit.

* Examine the log to determine the addresses of discovered devices.

* Convert the board addresses to decimal and edit the FxD device profiles to enter the I/O addresses into the PCI board forms.

6.	Configuring multiple Brooktrout boards that have the same number of channels

After running FxD, the FAXINIT program may be used to flash the lights of any Brooktrout board (including PCI boards) to determine which physical board uses which address.   The FAXINIT program flashes lights under Windows NT in the same way as the BTDRIVER program (described in the Brooktrout TR114 PCI Analog Hardware Guide) does under DOS.  (faxinit -h provides help for this utility.)

* In a MSDOS window, change to the Windows NT directory, then to the bfax subdirectory (eg, "cd \winnt\bfax").

* Run the program FAXINIT with the supplied address list.

faxinit addrs.lst

* Note that following the output of the PCI information is a list of individual channels and the address assigned to that channel.   For a given board address, find the channel that uses that address plus 4 (for example, to identify the board with address F800, find the channel using address F804).  Each channel has been given a device name of the form "fax0", "fax1", etc.

* Using the digits at the end of the fax name as a channel number, run faxinit as follows:

faxinit -f <chn>+<flash count>

For example, if the faxinit output included the line "fax4 f804 TR114+", then the command "faxinit -f 4+8"  would flash the lights 8 times on the board which was assigned address F800.   There must be no spaces between the two numbers.

7.	Notes time and NT server time should be set to the current time

FxD client and server systems must be configured so the current time is correct in both Notes and NT.  Otherwise, FxD may experience problems with logging, load balancing, and scheduled transmission.

Known Problems and Limitations

1.	"Fax To" field

If you receive the error message "no names found to send mail" when sending a fax, review the Fax for Domino R4 Help database comments on sending before saving.

2.	Deleting an incorrect entry from the "Fax To" field still populates the To field

If you send a fax with an incorrect entry in the Fax To field you will be prompted to supply a fax number for the incorrect entry.  If you choose to delete the entry rather than correct it, the fax will still be sent to the invalid address and you will receive a non-delivery report.

3.	Rendering issues

Notes radio buttons, checkboxes and twisties are not rendered.

4.	User receives misleading non-delivery report when cover page form is invalid

If you receive the error "could not convert cover page to fax format," make sure the cover page form used is in the FxD Cover Page database.

5.	Cannot print DBCS in fax page header

Brooktrout and GammaLink do not support DBCS in their headers.

6.	Device doc administration is not supported over the Web

You cannot create or edit device documents over the Web.

7.	RFX*.TIF  files in NFXTEMP directory

If a fax transmission is terminated before it is successfully received a RFX*.TIF file will remain in the NFXTEMP directory.  Check the directory periodically to clean up old files.

8.	FxD uninstall does not remove all files, subforms and notes.ini settings.

If you install FxD with combined administration and uninstall the product, the FxD subforms will remain in the Public Address Book.  Also, uninstall does not remove the FxD databases (spool, cover page, log), templates, and the Notes.ini file will contain $FAX FxD settings.