Canon LBP-8III Laser Beam Printer

User Notes for Level Zero Drivers

Lotus 1-2-3 Release 2 ALLWAYS

INTRODUCTION

Level Zero device drivers are software programs that enable the Canon

LBP-8III Laser Beam Printers to print text and graphs from Lotus 1-2-3

using ALLWAYS.

The drivers are licensed by Level Zero, Inc. solely for the LBP8-III

Laser Beam Printers manufactured by Canon, Inc., for use with ALLWAYS

and Lotus 1-2-3 Release 2.

DESCRIPTION

The Level Zero driver package for the LBP-8III consists of two program

files and an ASCII text file. The program files contain the ALLWAYS

drivers, CANON3XV.APD and CANON3X.API. The ASCII file, README.DOC,

contains a plain text copy of the User Notes.

The drivers support the Canon LBP-8III Laser Beam Printer when operat-

ed with ALLWAYS and Lotus 1-2-3 Release 2. In addition to the ALLWAYS

soft fonts, the driver makes available LBP-8III resident scalable

fonts in several font sizes and face characteristics. For 8.5 by 11

inch paper, the 1-2-3 page length setting should be 62.  Experimenta-

tion is recommended if you need to use a different paper length.

The Lotus International Character Set is accurately rendered by the

ALLWAYS soft fonts. The LBP-8III scalable fonts render all of the LICS

characters except four:

code        description        substitute

----  -----------------------  ----------

215   upper case OE dipthong        O

221   upper case Y umlaut           Y

225   lower case a acute            a

247   lower case oe ligature        o

Character print positioning of the LBP-8III scalable fonts is con-

trolled by the driver instead of the printer. This means that the

"backspace" function is not available for merge character printing

LICS characters or other symbol combinations with printer resident

scalable fonts.

The printer's resident scalable SWISS and DUTCH faces are each avail-

able in four point sizes (18,24, 36 and 48) with the following at-

tributes:

Upright and Italic style,

Normal and Bold weight,

Plain, Outline, Shadow Outline and Filled Shadow Outline

attributes.

This implementation of LBP-8III scalable fonts makes available for

text and graph printing a selection of faces/sizes/styles/weights/

attributes that are not otherwise available from ALLWAYS.

The ALLWAYS downloadable soft fonts are Courier, Times and Triumvirate

and are available in a variety of point sizes for upright and italic

styles in normal and bold weights.

The ALLWAYS driver prints graphs portrait and landscape orientations

in raster mode and uses 1-2-3's downloadable soft fonts for graph

text.

Text underlining of LBP-8III scalable fonts is not supported. In order

to underline, the user must place a line at the bottom of the cell(s)

to be underlined using the /Format Lines command.

DRIVER INSTALLATION

To install the LBP-8III ALLWAYS drivers, copy the two driver files

CANON3XV.APD and CANON3X.API from the distribution disk to your

ALLWAYS directory.

You will then be able to select the driver from among all of your

installed printer drivers right from ALLWAYS with the /Print Configur-

aion command.

RECOGNIZING AND SOLVING OPERATIONAL PROBLEMS

There are no ALLWAYS display fonts for the text to be printed with

LBP-8III scalable font attributes that are supported by this driver.

ALLWAYS will select the display font with the closest resemblance.

ALLWAYs will correctly represent upright, italic, normal and bold

characteristics. Some of the LBP-8III font sizes exceed the ALLWAYS

available screen font sizes and are displayed by the nearest available

screen size but the displayed line spacing and character pitch cor-

rectly reflect the actual size of the printed font.

If the user selects Outline, Shadow Outline or Shadow Outline Fill

text to print on a background that is shaded Solid, only the solid

shaded background is displayed. The desired text will print white-on-

black.

While a driver is in use the printer is temporarily placed into modes

that may not be supported by other programs. Should the computer get

disconnected or go down for any unplanned reason while printing the

printer will remain in the last mode set. To recover, the printer

should be reset with the button on the printer console or by manually

turning the printer power OFF and ON. If the printer goes down or

be reset for any reason while in 1-2-3 it is likely that the program

will have to be restarted.

In situations where a large number of soft fonts are down loaded to

the LBP-8III printer the error message "23 MEMORY FULL" may appear

on the printer's Message display along with an ALARM indication.

This means that the print session has reached the limit of the printer's

memory.  To recover from this situation press the "ERROR SKIP" and "ON

LINE" keys.  This will permit printing to continue, however, the font

used will be unpredicatable.

To avoid enountering this Memory Full situation the user should either

use fewer fonts in the print session or add additional memory to the

printer with a memory expansion board.

PRINTER SETUP

The Canon LBP-8III printer does not have DIP switches. As a minimum

the user may need to set up the type of interface (RS-232C serial or

Centronics parallel) using the front panel controls.  These are manda-

tory settings, covered below. The default factory interface setting is

Centronics.

The text and graphics drivers take advantage of the printers' remote

configurability to perform other set-up operations of a temporary

nature. Upon normal exit the drivers place the printer back into ISO

text mode.

MANDATORY SETTINGS

The following settings are required to match the interface:

GROUP           ITEM            VALUE

=====           ====            =====

Interface       INTERFACE       CENTRO or

RS-232C

If CENTRO (for Centronics parallel) is selected then the following

printer interface setting must be in force:

GROUP           ITEM             VALUE

=====           ====             =====

Interface       bsyack           A-B-A

This completes parallel interface settings. It is recommended that

they be saved to NVRAM.

If RS-232C is selected then the following parameters must also be

set:

GROUP           ITEM             VALUE

=====           ====             =====

Interface       baud             desired rate

Interface       rsmode           8S

Interface       dtr              Ready - H

Interface       Xon/Xoff         OFF

Interface      etx/ack           OFF

This completes serial interface settings. It is recommended that they

be saved to NVRAM.

OPTIONAL SETTINGS

Setup options:

Printer front panel message display language.

Paper length and number of copies.

Page orientation.

Options controlled by text driver (any selected

by user prior to  use of driver will be lost):

CR, LF, FF, VT are printed as sent (nothing added or implied).

###

(c) COPYRIGHT 1990, LEVEL ZERO, INC., ALL RIGHTS RESERVED

*LBP8-III Laser Beam Printers are products of Canon, Inc.

*Lotus 1-2-3 and ALLWAYS are products of Lotus Development Corp.

*Canon LBP-8III ALLWAYS driver (c) Copyright 1990, Level Zero, Inc.