Version 1.0

README

Tivoli Netcool Performance Manager for Wireless Technology Package

GSMGPRS Siemens BSS BR9 Patch 1.0.0.2

Note: Before using this information and the product it supports, read the information in Notices on page 10.

- Table of Contents

1	About this Documentation	1

1.1	Audience	1

1.2	Required Skills and Knowledge	1

2	Installation	2

2.1	Pre-requisite Requirement	2

2.2	Tech Pack Patch Contents	3

2.3	Patch Installation Procedure	3

3	Defects Fixed	8

4	Known Problems	9

Appendix A	Notices and Trademarks	10

# About this Documentation

## Audience

The target audience of this document is IBM Performance Manager for Wireless customers. They should be familiar with telecommunication and IT principles and should also have a good understanding of Solaris.

IMPORTANT: Before attempting an installation of Performance Manager for Wireless you are strongly advised to read the release notes and any readme files distributed with your Performance Manager for Wireless software. Readme files and release notes may contain information specific to your installation not contained in this guide. Failure to consult readme files and release notes may result in a corrupt, incomplete or failed installation.

Note: Performance Manager for Wireless Administrators should not, without prior consultation and agreement from IBM, make any changes to the Index Organized tables or database schema. Changes to the Index Organized tables or database schema may result in corruption of data and failure of the Performance Manager for Wireless System. This applies to all releases of Performance Manager for Wireless using all versions of interfaces.

## Required Skills and Knowledge

This guide assumes you are familiar with the following:

- General IT Principles

- Sun Solaris Operating System

- Oracle Database

- Windows operating systems

- Graphical User Interfaces

- Network Operator's OSS and BSS systems architecture

This guide also assumes that you are familiar with your company’s network and with procedures for configuring, monitoring, and solving problems on your network.

# Installation

## Pre-requisite Requirement

Before installing this patch, the following GOM and GOMlet, and base technology packs must be installed.

- vallent-vt-Neutral_Core_GOM-1.0.tar.gz

- vallent-vt-Neutral_GSM_BSS_NSS_GOM-1.0.tar.gz

- vallent-vt-Neutral_GPRS_UMTS_CN_GOM-1.0.tar.gz

- vallent-vt-Neutral_GPRS_BSS_GOM-1.0.tar.gz

- vallent-vt-Neutral_UMTS_UTRAN_GOM-1.0.tar.gz

- vallent-vt-VNL_GOMlet-2.0.tar.gz (or later version)

- vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.1.tar.gz

Note: vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.1.tar.gz is downloadable via http://www-01.ibm.com/support/docview.wss?uid=swg24020330

## Tech Pack Patch Contents

This section describes the contents of the technology pack package.

Docs (DOCS/)

- TNPMW_GSMGPRS_Siemens_BSS_BR9_IF0002_README.doc (this file)

- Quick reference guide for deployment of a Technology Pack Patch.

- PRS & FS

- Product Requirement Spec and Functional Spec for this Tech Pack Patch.

Tech Pack Package

- vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.2.tar.gz

- Contains patch files for loadmaps, main.xml, BusyHourInstance.xml and pre-installation.ksh.

Note: This Fixpack does not support any on site customization.

## Patch Installation Procedure

Install GSMGPRS Siemens BSS BR9 Tech Pack Patch as follows. This section explains the procedure to install this Technology Pack Patch on a step by step basis. Please read each section and sub-section carefully.

2.3.1 Tech Pack Uninstallation:

1) Skip this Step if SummaryInstance.xml and SummaryInstancePCalcs.xml in $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/summaries directory have not been provisioned. If SummaryInstance.xml and SummaryInstancePCalcs.xml have been provisioned, run the following commands to remove the provisioned summaries :

summary_admin -delete SIE_A_INTERFACE_

summary_admin -delete SIE_NSVCTHP_

summary_admin -delete SIE_CLEAR_MESSAGE#

summary_admin -delete SIE_RADIO_QUEUING#

summary_admin -delete SIE_CELL_INTRABSCHO_DR#

summary_admin -delete SIE_CCCH#

summary_admin -delete SIE_TBF#

summary_admin -delete SIE_CELL_INTERBSCHO_DR#

summary_admin -delete SIE_CELL_TCH_BH#

summary_admin -delete SIE_CELL_INTERBSCHO_F#

summary_admin -delete SIE_RADIO_RESOURCE#

summary_admin -delete SIE_BSSGP#

summary_admin -delete SIE_IMMASSIGN#

summary_admin -delete SIE_TCHTRAFFIC#

summary_admin -delete SIE_INTRACELLHO#

summary_admin -delete SIE_SDCCH#

summary_admin -delete SIE_RLC#

summary_admin -delete SIE_CELL_INTRABSCHO_DI#

summary_admin -delete SIE_STANTCHFR#

summary_admin -delete SIE_CELL_INTERSYSHO_S#

summary_admin -delete SIE_CELL_INTERBSCHO_DI#

summary_admin -delete SIE_CELL_INTERSYSHO_DI#

summary_admin -delete SIE_CELL_INTRABSCHO_F#

summary_admin -delete SIE_ASSGNT_SDCCH_TCH_FR#

summary_admin -delete SIE_CELL_INTERBSCHO_S#

summary_admin -delete SIE_CELL_RXLEVRXQUAL#

summary_admin -delete SIE_PROCESSOR#

summary_admin -delete SIE_CELL_INTERSYSHO_DR#

summary_admin -delete SIE_CELL_INTERSYSHO_F#

summary_admin -delete SIE_CELL_INTRABSCHO_S#

2) Skip this Step if BusyHourInstance.xml in $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/busyhours directory has not been provisioned. If BusyHourInstance.xml has been provisioned, run the following commands to remove provisioned busyhours :

sbh_admin -d –n Siemens_BS_Abis_data_traffic_Busy_Hour

sbh_admin -d –n Siemens_BS_Abis_speech_traffic_Busy_Hour

sbh_admin -d –n Siemens_BS_Processor_Load_Busy_Hour

sbh_admin -d –n Siemens_BSC_PCU_Load_Busy_Hour

sbh_admin -d –n Siemens_BSC_PDCH_Busy_Hour

sbh_admin -d –n Siemens_BSC_Traffic_FR_Busy_Hour

sbh_admin -d –n Siemens_BSC_Traffic_HR_Busy_Hour

sbh_admin -d –n Siemens_BSC_Traffic_Total_Busy_Hour

sbh_admin -d –n Siemens_Cell_RLC_Busy_Hour

sbh_admin -d –n Siemens_Cell_RLC_total_Busy_Hour

sbh_admin -d –n Siemens_Cell_SDCCH_Busy_Hour

sbh_admin -d –n Siemens_Cell_TCH_FR_Busy_Hour

sbh_admin -d –n Siemens_Cell_TCH_HR_Busy_Hour

sbh_admin -d –n Siemens_Cell_TCH_Total_Busy_Hour

3) Uninstall the tech pack using the following command:

techpack_admin -u techpack -n "Siemens GSMGPRS BSS"

4) Rename vallent-vt-GSMGPRS_Siemens_BSS_BR9.remove script to vallent-vt-GSMGPRS_Siemens_BSS_BR9.remove.backup in /appl/virtuo/admin/software

2.3.2 Tech Pack Installation:

5) Create Siemens_BSS_BR9 directory under $WMCROOT/admin/techpacks/packages

$WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9

mkdir -p $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9

6) Place tech pack patch, vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.2.tar.gz, into

$WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9 directory

7) Change current working directory to $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9 directory

cd $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9

8) Extract tech pack patch, vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.2.tar.gz in  $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9 directory

gunzip vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.2.tar.gz

tar xvf vallent-vt-GSMGPRS_Siemens_BSS_BR9-1.0.2.tar

9) Perform chmod pre-installation.ksh file in $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9 directory

chmod 755 $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9/pre-installation.ksh

10) Execute pre-installation.ksh file in $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9 directory

ksh $WMCROOT/admin/techpacks/packages/Siemens_BSS_BR9/pre-installation.ksh

11) Apply tech pack patch using the following command as per installation guide.

techpack_admin -a

12) Initialize the list of loaders in the system by executing series of command as follows:

sap stop

sapmgr stop

sapmgr start

sap start as

sap disp

13) Check the following loaders exist when sap disp command is issued :

loadsiemensgsmgprsbss_br8

loadsiemensgsmgprsbss_br9

loadsiemensgsmgprsbss_br91

14) Execute the following command to start the loader:

sap start loadsiemensgsmgprsbss_br8

sap start loadsiemensgsmgprsbss_br9

sap start loadsiemensgsmgprsbss_br91

15) To verify patch result, use the below command. The version tech pack for this patch should be listed upon issuing below command and expected output is version 1.0.2.

lcm_admin -list

16) Re-provision Data Availability files :

lcm_admin -load $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/loadmaps/dataavailability_gsmgprs_siemens_bss_br8.xml

lcm_admin -load $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/loadmaps/dataavailability_siemens_gsmgprs_bssbr9.xml

lcm_admin -load $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/loadmaps/dataavailability_siemens_gsmgprs_bssbr9.1.xml

17) Re-provision SummaryInstance.xml and SummaryInstancePCalcs.xml:

summary_admin -c -f      $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/summaries/SummaryInstance.xml

summary_admin -c -f $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/summaries/SummaryInstancePCalcs.xml

18) Re-provision BusyHours:

sbh_admin -i -f $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/busyhours/BusyHourInstance.xml -m overwrite

19) Applying the new Reports:

Go to the reports directory:

cd $WMCROOT/admin/techpacks/GSMGPRS_Siemens_BSS_BR9/1.0.1/reports

Run the following command:

report_impexp -i -u sysadm -p Sysadm01 -f . -l /Users/sysadm/ -dup overwrite

# Defects Fixed

Bugs resolved in GSMGPRS Siemens BSS BR9 fixpack version 1.0.0.2.

| CQ # (APAR #) | Headline |
| --- | --- |
| 70931 (IZ38846) | NPMW SIEMENS GSM BSS NA - SBH CANT BE LOADED |
| 45823 | Siemens GSMGPRS BSS BR9:- Project not using Module and Datasource correct convention |
| 42989 | GSMGPRS_Siemens_BSS_BR9 build1:- Incorrect mappping for "target_cell_vendor" column in nc_neighbour table |

# Known Problems

None.

Notices and Trademarks

This appendix contains the following:

- Notices

- Trademarks

#### Notices

This information was developed for products and services offered in the U.S.A.

IBM may not offer the products, services, or features discussed in this document in other countries. Consult your local IBM representative for information on the products and services currently available in your area. Any reference to an IBM product, program, or service is not intended to state or imply that only that IBM product, program, or service may be used. Any functionally equivalent product, program, or service that does not infringe any IBM intellectual property right may be used instead. However, it is the user's responsibility to evaluate and verify the operation of any non-IBM product, program, or service.

IBM may have patents or pending patent applications covering subject matter described in this document. The furnishing of this document does not grant you any license to these patents. You can send license inquiries, in writing, to:

IBM Director of Licensing
IBM Corporation
North Castle Drive
Armonk NY 10504-1785
U.S.A.

For license inquiries regarding double-byte (DBCS) information, contact the IBM Intellectual Property Department in your country or send inquiries, in writing, to:

IBM World Trade Asia Corporation
Licensing
2-31 Roppongi 3-chome
Minato-ku
Tokyo 106-0032
Japan.

The following paragraph does not apply to the United Kingdom or any other country where such provisions are inconsistent with local law: INTERNATIONAL BUSINESS MACHINES CORPORATION PROVIDES THIS PUBLICATION “AS IS” WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF NON-INFRINGEMENT, MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE. Some states do not allow disclaimer of express or implied warranties in certain transactions, therefore, this statement may not apply to you.

This information could include technical inaccuracies or typographical errors. Changes are periodically made to the information herein; these changes will be incorporated in new editions of the publication. IBM may make improvements and/or changes in the product(s) and/or the program(s) described in this publication at any time without notice.

Any references in this information to non-IBM Web sites are provided for convenience only and do not in any manner serve as an endorsement of those Web sites. The materials at those Web sites are not part of the materials for this IBM product and use of those Web sites is at your own risk.

IBM may use or distribute any of the information you supply in any way it believes appropriate without incurring any obligation to you.

Licensees of this program who wish to have information about it for the purpose of enabling: (i) the exchange of information between independently created programs and other programs (including this one) and (ii) the mutual use of the information which has been exchanged, should contact:

IBM Corporation
5300 Cork Airport Business Park
Kinsale Road
Cork
Ireland.

Such information may be available, subject to appropriate terms and conditions, including in some cases, payment of a fee.

The licensed program described in this document and all licensed material available for it are provided by IBM under terms of the IBM Customer Agreement, IBM International Program License Agreement or any equivalent agreement between us.

Information concerning non-IBM products was obtained from the suppliers of those products, their published announcements or other publicly available sources. IBM has not tested those products and cannot confirm the accuracy of performance, compatibility or any other claims related to non-IBM products. Questions on the capabilities of non-IBM products should be addressed to the suppliers of those products.

All statements regarding IBM's future direction or intent are subject to change or withdrawal without notice, and represent goals and objectives only.

This information contains examples of data and reports used in daily business operations. To illustrate them as completely as possible, the examples include the names of individuals, companies, brands, and products. All of these names are fictitious and any similarity to the names and addresses used by an actual business enterprise is entirely coincidental.

If you are viewing this information softcopy, the photographs and color illustrations may not appear.

#### Trademarks

IBM, IBM logo, Tivoli, and Netcool are trademarks of International Business Machines Corporation in the United States, other countries or both.

Adobe, the Adobe logo, PostScript, and the PostScript logo are either registered trademarks or trademarks of Adobe Systems Incorporated in the United States, and/or other countries.

Java and all Java-based trademarks and logos are trademarks of Sun Microsystems, Inc. in the United States, other countries, or both.

Microsoft, Windows, Windows NT, and the Windows logo are trademarks of Microsoft Corporation in the United States, other countries, or both.

UNIX is a registered trademark of The Open Group in the United States and other countries.

Intel and Pentium are trademarks or registered trademarks of Intel Corporation or its subsidiaries in the United States and other countries.

Other company, product or service names may be trademarks or service marks of others.

IBM

© Copyright IBM Corporation 2008

International Business Machines Corporation

5300 Cork Airport

Business Park

Kinsale Road

Cork

Ireland

Printed in the Republic of Ireland

All Rights Reserved

IBM, IBM logo, Tivoli, and Netcool are trademarks of International Business Machines Corporation in the United States, other countries or both.

Other company, product and service names may be trademarks or service marks of others.