1) Obtain IPMIUTIL from http://ipmiutil.sourceforge.net/FILES/archive/ipmiutil-2.9.1.msi and install it on a Windows workstation.

2) Go to the IMM web interface and go to IMM Management -> Users:

3) Create a user called ipmiuser and assign it a password.

4) Run C:\Program Files (x86)\sourceforge\ipmiutil>ipmiutil sel -d -N <IMM IP address> -U <USERID> -P <Password>

You should see something similar to the following:

C:\Program Files (x86)\sourceforge\ipmiutil>ipmiutil sel -d -N 9.5.66.41 -U ipmiuser -P Passw0rd

ipmiutil ver 2.91

isel: version 2.91

Opening lan connection to node 9.5.66.41 ...

Connecting to node  9.5.66.41

-- BMC version 2.00, IPMI version 2.0

SEL Ver 51 Support 0a, Size = 511 records (Used=3, Free=508)

ClearSEL: Log Cleared successfully

ipmiutil sel, completed successfully