IBM System Cluster 1350 factory settings

May, 2009

# The following tables show the factory settings for network addressing, user IDs, rack numbering, BladeCenter® unit and node numbering, I/O bay location numbering, and hostnames of the various IBM® System Cluster 1350™ components.

## Table 1: Network addressing and hostnames

| IP Address | Hostname | Component |
| --- | --- | --- |
| 172.20.0.1 | mgt.cluster.com | Management node eth0 (cluster vlan) |
| 172.30.0.1 | mgt1.cluster.com | Management node eth1 (management vlan) |
| 172.29.0.1 | bmc | Management node alias eth0:1 (cluster vlan ) |
| 172.29.101.1* | bmc001 | Node BMCs |
| 172.20.1.1 | storage001 | Storage nodes |
| 172.30.2.1* | triton01A | DS3000/DS4000 storage controllers (A, B) |
| 172.20.101.1 | node001 | Compute nodes |
| 172.40.101.1 | hca001 | HCA cards ib0 (cluster vlan) |
| 172.20.4.1 | user001 | User nodes |
| 172.30.10.1* | myri001 cis001 volt001 ql001 | Myrinet or InfiniBand: Voltaire 2004, 2012, 2036, 9096, 9288,4036, 4700, QLogic 9024, 9080, 9120, 9240, 12200, 12300, 12800 |
| 172.30.20.1* | ts001 | Terminal servers: Cyclades ACS32/ACS48 |
| 172.30.30.1* | rsa001, avocent001 | RSA or Avocent MP5300 |
| 172.30.40.1* | cisco2960-001 | 10/100 Cisco switches |
| 172.30.50.1* | bnt8000-001 bnt8100-001 brocb50c-001 brocb24x-001 brocfcx648 cisco3750-001 cisco4948-001 force50-001 mlnxis5030-001 smc8024-001 smc8848-001 smc8708-001 volt8500-001 | 1U/2U 1G and 10G Ethernet Switches Blade Network Technologies 8000, 8100, 8126 Brocade Networks Netiron ZB50C(CES2048CX), FCX648 (B48Y) Cisco 3750, 3560, 4900M, 4948 Force10 S50, S2410 Mellanox IS5030 SMC 8848M, 8708, 8024 Voltaire 8500 |
| 172.30.60.1* | apc001 | APC |
| 172.30.70.1* | rcm001 | RCM |
| 172.30.80.1* | brocb08r-001 brocb16r-001 cisco6509-001 force150-001 force300-001 force600-001 force1200-001 force600i-001 force1200i-001 mlnxis5200-001 mlnxis5600-001 | Enterprise Ethernet Switches Brocade B08R (BigIron RX-8-AC), B16R (BigIron RX-16-AC) Cisco 6509 Force10 E 600/E600i/E1200/E1200i and C150/C300 Mellanox IS5200 and IS6500 |
| 172.30.85.1* | fc001 | Fibre Channel switches |
| 172.30.101.131* | mm001 | BladeCenter management module |

* Indicates the hard-coded IP address of the various Cluster 1350 components.

## Table 2: Compute node IP addressing

| Rack name | Network | Rack number | Node* |
| --- | --- | --- | --- |
| Rack1 | 172.20 | 101 | 1 to 84 |
| Rack2 | 172.20 | 102 | 1 to 84 |

Example: rack 1, node 1 = 172.20.101.1

* Node numbering increases from the bottom of the rack upward.

* BladeCenter unit node numbering increases from the bottom of the rack upward and from left to right.

## Table 3: BMC IP addressing

| Rack name | Network | Rack number | Node |
| --- | --- | --- | --- |
| Rack1 | 172.29 | 101 | 1 to 84 |
| Rack2 | 172.29 | 102 | 1 to 84 |

Example: rack 1, node 1 = 172.29.101.1

## Table 4: BladeCenter IP Addressing BC-H 8852-xxx or 8677-xxx with an advanced management module (AMM)

| Network | Rack number | I/O bay location number (1-10) + BladeCenter unit number (1-6) |
| --- | --- | --- |
| 172.30* | 101 | 1 to 10 (SM I/O bay 1 - SM I/O bay 10) + 1 to 6 BladeCenter unit number |
| 172.30* | 102 | 1 to 10 (SM I/O bay 1 - SM I/O bay 10) + 1 to 6 BladeCenter unit number |
| 172.30* | 101 | Advanced management module* 130 + 1 to 6 BladeCenter unit number *Advanced management modules in the same BladeCenter unit share one IP address. |

Examples: 172.30.101.34 - the switch module fitted to I/O bay 3 in BladeCenter unit 4 of rack 1

172.30.102.46 - the switch module fitted to I/O bay 4 in BladeCenter unit 6 of rack 2

172.30.106.104 - the switch module fitted to I/O bay 10 in BladeCenter unit 4 of rack 6

172.30.106.133 - the AMM for BladeCenter unit 3 of rack 6

## Table 5: InfiniBand HCA IP Addressing

| Network | Rack number | Rack name | HCA* |
| --- | --- | --- | --- |
| 172.40 | 101 | Rack1 | 1 to 84 |
| 172.40 | 102 | Rack2 | 1 to 84 |

Example: rack 1, hca 1 = 172.40.101.1

*Host channel adapter (HCA) numbering increases from the bottom of the rack upward.

# Table 6: Factory set user IDs and passwords

| Hardware | User ID | Password | Comments |
| --- | --- | --- | --- |
| Avocent MP 5300 | root | avocent | Default |
| APC switch | apc | apc | Default |
| ITouch terminal | access | system | Default |
| InReach terminal | InReach | access | Default |
| BNT switch | admin | admin | Default |
| Brocade | none | none | Default |
| Cyclades terminal | root | tslinux | Default |
| Cisco switch | (see Comments) | ibm1350 | Press Enter for the userid prompt |
| SMC switch | admin | admin | Default |
| SMC 8024L2 | admin | smcadmin | Default |
| AStT | (see Comments) | infiniti | Press Enter for the userid prompt |
| Fibre Channel switch | admin | password | Default |
| Force10 | none | none | For priv exec, the password is force10 |
| RSAs | USERID | PASSW0RD | The 0 in passw0rd is a zero |
| BladeCenter management module | USERID | PASSW0RD | The 0 in passw0rd is a zero |
| Cisco InfiniBand switch | admin | adminpass | Default |
| Mellanox | admin | admin | Default |
| QLogic InfiniBand switch | admin | adminpass | Default |
| Voltaire root: | br6000 |  | Required for first log in (default) |
| Voltaire InfiniBand switch | admin | 123456 | Default |
| QLogic BC FC switch module | USERID | PASSW0RD | The 0 in passw0rd is a zero |

## Table 7: Operating system passwords

| Operating system | USERID | Password |
| --- | --- | --- |
| Management server | root | ibm1350 |
| Nodes | admin root | cluster cluster |

_________________________

IBM, BladeCenter and 1350 are trademarks of the IBM Corp. in the United States, other countries, or both. Other company, product, or service names may be trademarks or service names of others.

©Copyright International Business Machines Corporation 2007. All rights reserved.

US Government Users Restricted Rights – Use, duplication or disclosures restricted by GSA ADP Schedule Contract with IBM Corp.